Combined optics for PREx in 2010

Jay Benesch

Abstract

Combined optics, aka Moller optics to Ops, uses the two quad Moller polarimeter settings at all times.  MQA1H01 and MQA1H04 are used to bring the optics under control from the exit of the Moller polarimeter to the target and beam dump.  The solution shown is a slight modification of that created in June 2003.  The vertical spot size at the laser-electron interaction point in the Compton chicane was reduced to  = 60m from 100m including dispersive effects.

Optics

In June 2003 all of the combined (Moller) optics files for hall A were revised to use a two quad solution for the Moller polarimeter settings.  Optics were defined at 100 MeV intervals from 500 MeV to 6100 MeV and resulting files made available to the Magnet Scaling Tool (MST).  MST selects the closest file and scales it linearly to the actual beam energy.  PREx at 1056 MeV will therefore use the 1100 MeV file.  The plots below are taken from that file, slightly modified to reduce the vertical beam spot in the Compton chicane to  = 60m from 100m including dispersive effects.

Figure 1.  Hall A optics from Lambertson to pivot.  Note extreme horizontal (red) focusing by Moller system.  Pivot just before right edge of plot.

Figure 2.  Beam envelopes in hall A assuming perfect adiabatic damping.  0.05 cm vertical scale.  Blue is horizontal dispersive component assuming 1E-4 dp/p.  Black is vertical dispersive component.

Figure 3.  Phase advance between BPMs at end of line, ~141 and 147 m.  Pivot at 148.5m.  Vertical axis extent 90 degrees.

Figure 4.  Raster response.  Red, horizontal, is excited at 40% of full scale to get ~6mm half extent.  Green, vertical, is excited at 10% of full scale to get the same extent due to effect of quad MQA1H04.  Note that the Moller quads flip the sign of the horizontal raster with respect to excitation.  Further, the vertical fast raster excursion is only40 microns at the Moller target while the horizontal excursion is 2.5 mm.

First Conclusions

- Combined optics, needed for high current Moller polarimetry, is possible at 1.056 GeV for PREX.

- Spot size at target x~y~300m

- Spot size at Compton interaction point x~0m, y~60m including dispersion

- Raster settings are 4:1 asymmetric to get a square at the pivot.  With the low energy, a large raster may be obtained in both planes at the pivot target.  This raster may be too small at the Moller ploarimeter target.

- Phase advance between the last two BPMs is minimal.  The utility of simultaneous excitation of two modulation coils must be examined for this optics.

The material above was presented to R. Michaels.  The raster pattern at the Moller target is not adequate, as hypothesized above.  MQA1H04 was turned off to allow higher vertical raster and lower horizontal raster at the Moller target.  Size is now 1.5mm x 0.5 mm, as shown in figure 5.

Figure 5.  Raster to Moller target, which is at right axis.  2mm vertical scale.  Red horizontal, greem vertical

Figure 6.  Raster to physics target.  Vertical scale +- 5mm.  Same excitation as figure 5.

Figure 7.  Phse advance in last 10m with altered optics

Figure 8.  Beta functions from end of Compton to pivot

Figure 9.  Beam envelopes to pivot.  400 microns X by 150 microns Y at pivot.  Compare to figure 2.

Figure 10.  Beam envelope to beam dump.  2mm vertical scale

Second Conclusions

- Raster settings are 1.75:1 asymmetric to get a square at the pivot.

- Raster adequate at Moller target.  Desired 0.7mm square.  Actual 1.5mm by 0.5mm

- Beam is adequately contained to beam dump

- Spot size at pivot is asymmetric.  The accelerator could run this was or could change one quad (MQA1H04) and the raster settings to switch between production running and Moller polarimetry.