|  | Task Hazard Analysis (THA) Worksheet (See ES&H Manual Chapter 3210 Appendix T1 Work Planning, Control, and Authorization Procedure) | Click For Word Doc |
| --- | --- | --- |

| Author: | Tyler Lemon |  |  |  |  | Date: |  | 5/26/2016 |  |  | Task #: If applicable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Complete all information. Use as many sheets as necessary |  |  |  |  |  |  |  |  |  |  |  |  |
| Task Title: |  | RICH electronics cooling system and N2 purge system |  |  |  |  |  |  | Task Location: | Hall B |  |  |
| Division: |  |  |  |  | Department: |  |  |  |  | Frequency of use: |  |  |
| Lead Worker: |  |  |  |  |  |  |  |  |  |  |  |  |
| Mitigation already in place: Standard Protecting Measures Work Control Documents |  |  |  |  |  |  |  |  |  |  |  |  |

| Sequence of Task Steps | Task Steps/Potential Hazards | Consequence Level | Probability Level | Risk Code (before mitigation) | Proposed Mitigation (Required for Risk Code >2) | Safety Procedures/ Practices/Controls/Training | Risk Code (after mitigation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Potential fire hazard to detector equipment due to heat load produced by power supplies. | H | L | 3 | Use a combination of cooling and interlock systems which will shut down any power supplies producing heat in the system when a fault occurs. Please see attached cooling and interlock system document. | Follow all JLAB standards and practices as presented in the JLAB ES&H manual. Standard interlock procedures will be used to mitigate any fire/electrical hazards per JLAB ES&H manual. All interlocks will be designed to fail in a safe mode. All individuals will be trained by lab standard. | 1 |
|  | RICH uses aerogel to produce Cherenkov light. Aerogel will absorb moisture out of the air. If the aerogel absorbs moisture, it will be detrimental to detector performance. | L | M | 2 | To lower the humidity in the detector and prevent the aerogel from absorbing moisture, the detector body will be purged with nitrogen. | The nitrogen purge system will use the Hall B boil-off nitrogen supply. See attached document by George Jacobs for a system diagram. | 1 |

| Highest Risk Code before Mitigation: | 3 | Highest Risk Code after Mitigation: | 1 |
| --- | --- | --- | --- |

When completed, if the analysis indicates that the Risk Code before mitigation for any steps is “medium” or higher (RC≥3), then a formal Work Control Document (WCD) is developed for the task.  Attach this completed Task Hazard Analysis Worksheet.  Have the package reviewed and approved prior to beginning work.  (See ES&H Manual Chapter 3310 Operational Safety Procedure Program.)

Form Revision Summary Periodic Review – 08/13/15 – No changes per TPOC Revision 0.1 – 06/19/12 - Triennial Review. Update to format. Revision 0.0 – 10/05/09 – Written to document current laboratory operational procedure. ISSUING AUTHORITY; TECHNICAL POINT-OF-CONTACT; APPROVAL DATE; REVIEW DATE; REV.. ESH&Q Division; Harry Fanning; 08/13/15; 08/13/18; 0.1. This document is controlled as an on line file. It may be printed but the print copy is not a controlled document. It is the user’s responsibility to ensure that the document is the same revision as the current on line file. This copy was printed on 7/30/2026.