|  |  | TITLE: | ES&H Manual |
| --- | --- | --- | --- |
| DOCUMENT ID: | 6132 Appendix T3 Ladder Use on Scaffold or Elevated Platform |  |  |

DRAFT

- Purpose

When a scaffold or other elevated work platform height is not adequate for a worker to safely reach a work area, the preferred method for elevating the worker is to add an additional level to the structure.  When this is not feasible, Jefferson Lab allows a ladder to be used on the platform but only when specific requirements are met.  This appendix provides these requirements.

- Scope

This appendix applies when a worker is on an elevated platform and is, by any means, elevated above the passive fall protection (i.e.: guardrails) system provided.  (See ES&H Manual Chapter 6131 Fall Protection Systems for more information on passive fall protection.)

The process steps for this procedure are performed in coordination with ES&H Manual Chapter 6132 Ladder and Scaffold Safety.

- Responsibilities

NOTE:	Management authority may be delegated at the discretion of the responsible manager.

  - Ladder/Scaffold Worker

- Complete SAF 307 Ladder Safety training before using a ladder on an elevated platform.

- Obtain authorization from the Competent Person – Scaffold or Supervisor prior to using a ladder on an elevated platform.

- Complete scaffold training (SAF 303 – Scaffold Safety) before using a ladder on a scaffold structure.

  - Competent Person – Scaffold/Supervisor

- Prior to authorizing the use of a ladder on a scaffold or elevated platform ensure an alternate method (e.g.: additional level, man-lift) is not feasible.

- Ensure platform setup meets fall protection requirements.  (See also: ES&H Manual Chapter 6131 Fall Protection System.)

- Requirements

Unless otherwise stated only trained and qualified Ladder Workers may perform the process steps listed below.

  - Use of Ladder on Elevated Platform Requirements

- Examine the Platform:

- For All Structures: Ensure the passive fall protection (i.e.: guardrail) system on the working level of the platform meets or exceeds OSHA requirements; and all sides of the platform are protected.  (Contact the Competent Person – Scaffold or Supervisor if system in inadequate.)

- For Scaffold Structure – Ensure the structure has been planned, selected, erected, and inspected in accordance with the requirements of ES&H Manual Chapter 6132 Appendix T2 Scaffolds.

- Inspect Ladder Equipment and Ensure Proper Use

- Inspect the ladder to be used in accordance with the requirements of ES&H Manual Chapter 6132 Appendix T1 Ladders.

- Ensure the “Highest Standing Level” (designated on the ladder’s Duty Rating Sticker) is adequate for the work.

- Set the Ladder Up

- Position the ladder at the location desired.

- Measure the distance from the ladder’s base to the nearest open platform edge.

| IF | THEN |
| --- | --- |
| The distance from the ladder base to the nearest open platform edge (D1) is less than the ladder’s highest standing level (D2) plus 6 feet. | Competent Person – Scaffold/Supervisor: - Increase the top rail height of the guard rail to a distance adequate for the ladder’s highest standing level (D2). OR - Incorporate another method of fall protection. (See ES&H Manual Chapter 6131 Fall Protection Systems.) |
| The distance from the ladder base to the nearest open platform edge (D1) is greater than or equal to ladder’s highest standing level (D2) plus 6 feet. | Guardrail system, as designed, is considered adequate fall protection. |
| Figure 1: Ladder on Elevated Platform |  |

- References

In all cases, when a ladder is used on a scaffold platform, the following mitigation is required in accordance with “29 CFR 1926 Subpart L 451(f)(15):

- When the ladder is placed against a structure which is not a part of the scaffold, the scaffold shall be secured against the sideways thrust exerted by the ladder;

- The platform units shall be secured to the scaffold to prevent their movement;

- The ladder legs shall be on the same platform or other means shall be provided to stabilize the ladder against unequal platform deflection, and

- The ladder legs shall be secured to prevent them from slipping or being pushed off the platform.”

- Revision Summary

Revision 0 – ##/##/## – This is new content.

|  | ISSUING AUTHORITY | TECHNICAL POINT-OF-CONTACT | APPROVAL DATE | EXPIRATION DATE | REV. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ESH&Q Division | Ned Walker |  |  | 0 |  |

This document is controlled as an on-line file.  It may be printed but the print copy is not a controlled document.  It is the user’s responsibility to ensure that the document is the same revision as the current on line file.  This copy was printed on 7/31/2026.

Scaffold training is not required when using a ladder on a permanent elevated platform structure.  These structures include but are not limited to: catwalks, decks, elevated open walkway, etc.