|  | Scaffold Inspection Checklist (See ES&H Manual Chapter 6132 Appendix T2 Scaffold for more information) |  |
| --- | --- | --- |

| Project |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User: |  |  |  |  |  | Location: |  |  |  |  |  |  |  |  |  |
| For all Scaffolds Structures: |  |  |  |  |  | Yes |  | No |  |  | N/A |  |  | Action/Comments |  |
| - Is the structure appropriate for the work? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is the structure level and plumb? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is the structure capable of carrying intended loads? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Components? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - In safe condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Placed properly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Compatible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Planking or scaffold boards– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Rated for scaffold use? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 6” to 12” overlap beyond supports? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Secured to prevent displacement or uplift? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Working platforms – Fully planked? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Guard Railing – Properly placed? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Access – Stair, ladders, rail gates? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Bracing – at the base and intermediate levels? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Toeboards – Provided and properly installed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Assess the following hazardous conditions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Power lines? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Wind loading? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Footing integrity? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scaffold is OK to Use: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |
| Rolling Tower Scaffold |  |  |  |  | Yes |  | No |  |  | N/A |  |  | Action/Comments |  |  |
| - Tower height <4 times the minimum base width? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is tower fully braced on both sides? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Screw jacks extended <12 inches? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Casters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Proper size? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Roll freely and lock securely? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Secured to frame? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tubular Frame Scaffold |  |  |  |  | Yes |  | No |  |  | N/A |  |  | Action/Comments |  |  |
| - Are sections properly placed and adequate size? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Has scaffold been tied to structure at least every 30’ in length and 26’ in height? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Is overhead protection or wire screening in good condition and adequate? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Screw jacks adjusted correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Competent Person’s Signature: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Print Name |  |  |  |  |  |  |  |  | Contact # |  |  |  |  |  |  |
| Comments: (Including approved temporary alterations ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Upon completion attach this form to scaffold.