|  |  | TITLE: | ES&H Manual |
| --- | --- | --- | --- |
| DOCUMENT ID: | 6660 Appendix T1 Transport of Hazardous Material |  |  |

DRAFT

- Purpose

Jefferson Lab allows limited quantities of hazardous material to be transferred from one location to another (i.e.: on-site), and shipped (i.e.: off-site) in accordance with identified safety standards.  This appendix provides the process steps used to ensure appropriate safety precautions are utilized during these operations.

- Scope

This appendix applies to hazardous material in quantities at or below “Allowable On-site Transfer Limits” (as noted below).  Only Jefferson Lab’s Industrial Hygiene Department, Radiation Control Department, or Shipping and Receiving Department can authorize hazardous material off-site shipment, or on-site transport in quantities exceeding these limits.

The following hazardous material has additional requirements:

- Radioactive Material – See ES&H Manual Chapter 6310 Protection from Ionizing Radiation

- Medical Waste – See ES&H Manual Chapter 6800 Appendix T4 Regulated Medical Waste Management

The process steps for this procedure are performed in coordination with ES&H Manual Chapter 6660 Hazardous Waste Management (Link to be established upon approval of document.)

- Responsibilities

NOTE:	Management authority may be delegated at the discretion of the responsible manager.

  - Anyone at Jefferson Lab

- When transporting hazardous material on-site follow the process steps provided in ES&H Manual Chapter 6660 Appendix T1 Transportation of Hazardous Material (Link to be established upon approval of document).

- Request hazardous material identification and support from Industrial Hygiene as required.

  - Supervisor/Subcontracting Officer’s Technical Representative (SOTR)/Sponsor

- Ensure only qualified and appropriately trained individuals are allowed to transfer hazardous material on-site and ship hazardous material off-site.

    - SOTRs Also:

- Providing Jefferson Lab’s transportation requirements to subcontractors.

  - Hazardous Material Shipper

- Maintain appropriate training (SAF305 DOT Hazardous Materials Shipping) and medical monitoring, as required.

  - Industrial Hygiene

- Provide hazardous material identification and support as required.

  - Property Manager (i.e.: Hazardous Materials Transportation Officer)

- Ensure only qualified and appropriately trained individuals are allowed to ship hazardous material off-site.

- Review all work control documents dealing with hazardous material on-site transfer or off-site shipping.

- Ensure compliance with on-site transfer and off-site shipping hazardous materials rules and regulations.

- Process Steps

Unless otherwise stated, anyone at Jefferson Lab may perform the process steps outlined below.

  - On-Site Transfer of Hazardous Material:

- Prepare the transfer location; ensure it is appropriate in accordance with hazardous material requirements.  (See ES&H Manual Chapter 6610 Appendix T3 Chemical Labeling, Storing, and Transporting Requirements.)

- Identify the Hazardous Material and Quantity

    - Look at the container’s label – identify the material and its associated hazard class.

      - If the hazard class is not shown on the label – Use Jefferson Lab’s Material Safety Data Sheet Database.  The Hazard Class is generally listed under “Transport Information,” or similar, within “MSDS Contents” section.)

    - Estimate how full the container is to determine the quantity to be transferred or shipped.  (Example: if container holds 50 gallons and is half full, estimated quantity would be 25 gallons.)

- Review the Transportation Requirements for the Identified Hazard Class –

The table below provides examples of Jefferson Lab’s most common hazardous material.  If your hazardous material is not listed, use its associated Hazard Class to determine its requirements.

| Examples of Common Chemicals | DOT Hazard Class | Allowable On-Site Transfer Limit | Requirements |
| --- | --- | --- | --- |
| None - Jefferson Lab does not accept any Class 1 Material without pre-approval of the Jefferson Lab Site Office. | Hazard Class 1 – Explosives | Not Allowed | This type of material is considered to be a Risk Code >2. An Operational Safety Procedure is required in accordance with ES&H Manual Chapter 3210 Work Planning, Control, and Authorization Process. |
| - Helium, - Nitrogen, - Oxygen, - Acetylene - Cryogens | Hazard Class 2 – Gases Including: - Gas: Non-Flammable, Flammable, Toxic Gas, Oxygen, Inhalation Hazard, - Liquefied: Non-Flammable and Non-Poisonous | - Cylinder size not to exceed 220 lbs and total weight of load not to exceed 440 lbs - Toxics: Contact Industrial Hygiene | - Cap in place. Valve closed. - Proper Labeling. - Container is designed to handle specific material. - Transport only in open bed-trucks or forklifts. - Container secured in/on vehicle. - Appropriate PPE used during container handling. |
| - Poisonous or Flammable Cryogenic Gas: On-site transfer of this type of material is considered to be a Risk Code >2 and requires an Operational Safety Procedure in accordance with ES&H Manual Chapter 3210 Work Planning, Control, and Authorization Process |  |  |  |
| - Paint, - Solvents, - Adhesives, - Resins | Hazard Class 3 - Flammable Liquids | - Up to 8 gallons | - Close container and secured in/on vehicle. - No smoking - Appropriate PPE used during container handling |
| - Activated carbon - Lithium - Magnesium powder - Tungsten powder | Hazard Class 4 – Flammable Solids (Flammable Solid, Spontaneously Combustible, Dangerous When Wet) | - Up to 66 lbs. - When Wet: limit 1 oz - VERY DANGEROUS | - Close container and secured in/on vehicle. - No smoking - Appropriate PPE used during container handling |
| - Hydrogen peroxide - Epoxy hardeners or curing agents | Hazard Class 5 – Oxidizer and Organic Peroxide | - Up to 66 lbs. | - Close container and secured in/on vehicle. - No smoking - Appropriate PPE used during container handling |
| - Cyanide solutions - Infectious agents | Hazard Class – Toxic/Poisonous and Infectious Substances (PG III, Inhalation Hazard, Poison, Toxic) | - Contact IH | - Contact IH |
| - Activated equipment - Radioactive sources | Hazard Class 7 – Radioactive (I, II, III, and Fissile) | - Contact RadCon | - Contact RadCon |
| - Hydrofluoric acid - Sulfuric acid - Safe-D-Scale acid | Hazard Class 8 – Corrosive | - Up to 66 lbs. | - Close container and secured in/on vehicle. - Appropriate PPE used during container handling |
| - Lead debris - Asbestos (friable) | Hazard Class 9 – Miscellaneous Dangerous Goods | - Up to 66 lbs. | - Close container and secured in/on vehicle. - Appropriate PPE used during container handling |

- Transfer Hazardous Material

- Allowable Quantity: Ensure quantity is within the allowable transfer limit

If the allowable transfer limit is exceeded it is considered to be a Risk Code >2 and requires a work control document (Operational Safety Procedure) in accordance with ES&H Manual Chapter 3210 Work Planning, Control, and Authorization Process.) Only Jefferson Lab’s Industrial Hygiene Department, Radiation Control Department, or Shipping and Receiving Department can authorize hazardous material shipment off-site, or transport on-site in quantities exceeding these limits.

- Load container in a safe and responsible manner in accordance with requirements.

- Secure container, to prevent shifting during transfer in accordance with requirements.

- Deliver container to transfer location.  If applicable, ensure ownership is recognized and approved.

- Store material in prepared location, according to requirements.

  - Off-Site Shipping Requirements

Only Jefferson Lab’s Industrial Hygiene, Radiation Control, or Shipping and Receiving Departments can authorize hazardous material for shipment off-site.

When hazardous shipment is required provide the following information to the Industrial Hygiene, Radiation Control, or Shipping and Receiving Departments:

- Responsible Owner’s Contact Information

- Material Name

- Form of Material (i.e. liquid, solid, gas etc)

- Quantity

- Chemical Abstracts Service (CAS) number (if any)

- Material Safety Data Sheet (URL from Database is adequate)

- Material Identification Number (if any)

- Shipping Destination (Name, Address, and Telephone Number)

- Special Shipping Requirements

- Definitions

For the purpose of this document:

On-Site Transfer: refers to the movement of hazardous material from one area to another; when the material and both areas are under the authorization of Jefferson Lab.

Off-Site Shipment: refers to when hazardous material, under the authority of Jefferson Lab, is moved to an area that is not under the authority of Jefferson Lab.

- References

Jefferson Lab’s Hazardous Materials Transportation Policy and Procedures

49 CFR Transportation, Subchapter C - Hazardous Material Regulations Part 173 – 180

DOE O 460.1C, Packaging and Transportation Safety

- Revision Summary

Revision 0 – ##/##/## – This is new material.

|  | ISSUING AUTHORITY | TECHNICAL POINT-OF-CONTACT | APPROVAL DATE | EXPIRATION DATE | REV. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ESH&Q Division | Carl Iannacone |  |  | 0 |  |

This document is controlled as an on-line file.  It may be printed but the print copy is not a controlled document.  It is the user’s responsibility to ensure that the document is the same revision as the current on line file.  This copy was printed on 7/30/2026.

For the purposes of this document the term “on-site transfer” refers to the movement of hazardous material from one area to another; when the material and both areas are under the authorization of Jefferson Lab.

For the purposes of this document the term “off-site shipment” refers to the movement of hazardous material, under the authority of Jefferson Lab, to an area not under the authority of Jefferson Lab.