|  |  | TITLE: | ES&H Manual |
| --- | --- | --- | --- |
| DOCUMENT ID: | 6420 Appendix T2 Operational Safety Procedure Development for Qualifying Static Magnetic Fields |  |  |

DRAFT

- Purpose

When a Task Hazard Analysis indicates that a specialty magnets poses an unmitigated Risk Code>2 a work control document (WCD) is required to ensure hazard issues are addressed and appropriately mitigated.  This appendix provides the requirements for this WCD.

- Scope

This procedure provides the minimum safety requirements for specialized magnets with the potential power of:

- 60 mT (600 G) or more – measured at expected employee torso distance, or

- 600 mT (6000 G) or more – measured at expected employee extremity distance

- 5 Gauss Field – under certain conditions.

It does not preclude the need to incorporate additional mitigation to ensure the safety of people, equipment, and the environment from associated activities.

The process steps for this procedure are performed in coordination with ES&H Manual Chapter 6420 Non-Ionizing Radiant Energy (i.e.: Radio Frequency (RF), Microwave, and Electro/Static Magnetic) (Link to be established upon approval of document.); and ES&H Manual Chapter 3310 Operational Safety Procedure Program.

- Responsibilities

NOTE:	Management authority may be delegated at the discretion of the responsible manager.

  - Magnet Owner

- Complete a Work Control Document for magnets under your authority that have been determined to be a Risk Code >2.

- Process Steps

Unless otherwise stated the Magnet Owner performs the following process steps.

- Task Hazard Analysis

- Use ES&H Manual Chapter 3210 Appendix T2 Task Hazard Analysis (THA) Worksheet to outline the task steps or services required for the area/equipment.

- Identify known and potentially hazards conditions, suspected causes, and environmental concerns.

- Identify required Safety Procedures/Practices/Controls/Training.

- Operational Safety Procedure (OSP)

- Use ES&H Manual Chapter 3310 Operational Safety Procedure Program to develop an OSP to document the hazard issues and mitigation requirements associated with magnets.

- The following additional information is required to be addressed prior to review and approval of the document.

| Personnel | - Medical requirements - Incidental worker requirements |
| --- | --- |
| Magnetic Device Inventory | - Specifications - Define all components |
| Magnet Environment | - Exclusion zone for electronic equipment - Magnetic field survey and map - Ferromagnetic tool hazard area - Optical systems |
|  | Superconducting Magnets: - Cryogenic equipment |
| Procedures | - Normal use - Ferromagnetic object/tool sweep - Maintenance procedures - Off-normal and emergency procedures (i.e. quench) - LO/TO procedures |
| Controls | - Protective circuitry (quench) - Grounding/Ground-fault circuit-interrupter (GFCI) - Lead covers - Barricades - Warning signs - Beacons |
|  | Vacuum Systems - Window protection |
|  | Superconducting Magnets: - Pressure relief - Ventilation - ODH assessment - Quench protection |

- Completion of OSP Document

- Complete the process steps as outlined within ES&H Manual Chapter 3310 Appendix T1 Operational Safety Procedure (OSP) and Temporary OSP Procedure for validation, approval, recordkeeping and implementation.

- Revision Summary

Revision 1 – ##/##/## – Updated to reflect current laboratory operations.

|  | ISSUING AUTHORITY | TECHNICAL POINT-OF-CONTACT | APPROVAL DATE | EXPIRATION DATE | REV. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ESH&Q Division | Jennifer Williams |  |  | 1 |  |

This document is controlled as an on-line file.  It may be printed but the print copy is not a controlled document.  It is the user’s responsibility to ensure that the document is the same revision as the current on line file.  This copy was printed on 7/28/2026.