# FFA@CEBAF Working Group|Minutes

## Meeting date | time 6/3/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Alex C, Kirsten, Jay, Scott, Stephen, Kitty, Vasiliy, Dejan, Andrei, Todd |
| --- | --- |

# Intro Discussion

# Agenda topics

## Time allotted | 25 minutes | Agenda topic Adiabatic Arcs | Presenter Vasiliy/Randy

- Vasiliy’s poster:

  - High-power hadron accelerator which uses FFA arcs

    - Can compensate with more passes

    - Mostly conceptual

    - It’s the first and only publication of the concept so far

  - Application to CEBAF energy doubling

    - Chose 5+3 option, West Arc

    - Used Stephen’s data, made FFA cell using Mad-X (consistent with BMAD)

    - Triplet cells

    - 26 cells used

    - Offsets and dispersions go down while beta steady

  - Jacobian solver in Mad-X – but didn’t like

  - Started again with clean slate

  - Same FODO, scaled focal strength

    - Scaled quads in triplets up by same values

  - Polynomial reduces space between triplets

  - Ran optimizer again, ok

    - If combine sections 1 and 2 should be ok

    - Initial guess was pretty close to the right answer

  - More work to be done, but individual betas behaving decently

- Not on poster:

    - Phase advance per cell

      - Scott: This is reflecting the beta beating – in some ways it’s artificial, and accumulated phase advance would do a better job of showing what’s going on

      - Jumps not synchronized (in top plot) – shows that it’s doing the right thing in some sense

      - Spikiness in phase advance confuses the issue

      - There’s a redundancy in our linear variables. Phase advance is not some absolute number until you go into a truly periodic solution

      - If you pick a different set of initial beta functions, that whole spiky plot will change completely. The top plot will look a bit different, but the beginning/end would be the same. You’ll get the general trend

    - Looks regular

- How well does this cope with magnet errors (non-adiabatic)?

  - 1% error? Is there a correction algorithm?

  - Will need lots of diagnostics in this section.

- Having a long period section gives more freedom to control the phase advance.

- Scott: When matching at beginning and end, it’s beta that you care about the most, not phase advance

  - If only 1 plane, that’s true

  - If more than 1 plane, it’s tied to phase advance – have to have right phase advance

  - Middle cells don’t need to maintain phase advance.

  - Let phase advance float in the middle, and keep betas controlled – random idea

- Kirsten: this is transition from repeating cell that matches into linac optics?

  - Yes

  - Not accounting for TOF or spreaders optics yet.

  - Idea of this is to instead of having 4 splitters in the machine, use 2 on one side, and other side with a pair of these

    - IF we can get TOF and R56 in one spot, then adiabatic match in other side of CEBAF

- Need to take away higher energies – important to understand how we separate them in easiest possible way

  - Need to do TOF and R56

  - Better than CBETA here b/c 180-degrees (so maybe half to a third of CBETA TOF)

  - R56 is the biggest challenge

    - Must be connected to a chicane where you can adjust dispersion function as well

    - Need some quads too

    - Maybe each chicane works for several energies (3 maybe too much…2 maybe)

      - Can still add more dipoles for further separation

  - First attempt should be traditional chicane

    - More clever options might be less efficient – so check if it’s possible first

- Adding in splitters can also drop down the beta – might make arc matching easier

  - Add more quads

- Try to work with 2 sets of splitters to correct all of the TOF and R56, use adiabatic matching in other 2 spots.

- Jay – do we need to look at 3 passes through the injector (860 instead of 650 MeV) – would allow closer to a drift

  - 650 MeV was a minimum – but to make Dejan’s suggestion work, couldn’t we have a higher injection energy – then whatever we do with linac optics might work easier

    - For a drift linac (with no quads), it doesn’t really matter. Will look like a parabola

    - Will have a height of 250 m beta – too big.

  - Sparse quads?

    - The quads will mess up beta at lower passes and be invisible to higher

- We need to rethink the multipass optics in the linacs. Need to be revisited because the initial solution used very compact permanent magnets

  - We need to be able to scale the energy – won’t always be 1.2 GeV per linac

    - Will need electromagnets

- Kirsten: need a conversation about ARC diagnostics that we’ll need in the CEBAF FFAs

  - CEBAF may need more useful diagnostics

  - SL monitors, etc…

    - Moveable camera?

- Back to linac optics:

  - 9.6 m periodicity

    - Scale triplet strength as we go up in E

    - Still enough phase advance, getting 80 m beta with periodic structure

    - This is input into Vasiliy’s matching.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 minutes | Agenda topic IPAC22 | Presenter All

- Vasiliy’s IPAC contribution noted in the previous agenda section

- Ryan will print and carry Vasiliy’s paper, and present it at IPAC.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 minutes | Agenda topic AOB | Presenter All

  - Peak dispersion lowered from 4 to 2.7 m

  - Realized unused quad strength

    - Got it down to 1.8 m

  - Horizontal emittances are much larger than vertical

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g