# FFA@CEBAF Working Group|Minutes

## Meeting date | time 7/8/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Dejan, Kitty, Amy, Randy, Scott, Stephen, Kirsten, Vasiliy, |
| --- | --- |

# Intro Discussion

Dejan talking about changing the linacs…maybe more like ERHIC. Dejan uploaded something into the July 8th folder on the sharepoint. See AOB section for more.

# Agenda topics

## Time allotted | 25 minutes | Agenda topic Baseline Questions | Presenter Alex

- New question from Stephen: where does FFA end, at A/B/C, or one more half to D?

  - Would end on West side, go through one more NL pass to D

  - Will get 8.5, 9.5, or 10.5 passes.

  - Old design from Stephen had 7 passes on E and 6 on W

    - 5 on both sides the new way would be fine.

    - 6 on both sides doesn’t account for SR loss or energy tunability.

    - Assume 1050-1150 MeV/linac, with nominal of 1100 MeV/linac

      - We currently struggle with 1090 MeV/linac right now.

- Scott: we’ll need to pay attention to the degradation of the linacs over time, even after the refurbishment.

  - They want to refurb mostly C-100s

- Maybe we can get a presentation from SRF to talk about this?

  - 1050-1150 has been “checked” with Jay and Bob Rimmer already.

- Scott: We need to base lowest energy on worst-case energy.

  - Do need to make sure we are maintaining energy ratios for spreaders/splitters as well.

- Ryan: what about degradation of permanent magnets along with the linacs?

  - Stephen: depends on radiation levels – can tell if have that data.

  - Scott: definitely have issues with wigglers losing strength (lead end usually) at the 3% level

  - Stephen: also depends on material grades

    - We can adjust magnet strength with cooling water as well.

- Scott: we should be very conservative.

- Stephen: The reason we had 2 FFAs with 925-1090 range, was hard. If you have a conservative lower and not-so-conservative high end, we’ll need 2 FFAs again.

- Scott: Another case for 2 FFAs:

  - If you have 2 FFAs, the splitters end up being more realistic. Fewer lines, less complication, less horizontal real estate to manage. Maybe better for R56 too.

  - Working with smaller E range, and more optimized for SR.

  - The single FFA – not really that difficult in grand scheme. Having 6 splitter lines without a large energy range is challenging.

- Depends on users. If 22 GeV is enough, maybe 1 FFA. If they want more, then 2 might be smarter.

- If you go to two FFAs, you can make the magnets larger and safer with lower field.

  - Stephen likes 4 + 5 to get over 20, then have option of adding another later for further upgrades.

- Alex: cost and economy.

- Must finish Spreaders and Splitters for complete design

  - Ryan is working on this as a green-field re-design.

| Action Items | Person responsible | Deadline |
| --- | --- | --- |
| New Linac idea | Alex B |  |
| Spreaders | Ryan |  |

## Time allotted | 25 minutes | Agenda topic Single FFA Scheme | Presenter Dejan

  - We have 2 linacs, one coming with 650 MeV energy, up to 1.75 GeV. SL starts at 1.75 GeV.

  - Tune per cell (9.6 m between triplets) is going from ~0.06 to 0.38.

  - Assumed highest possible gradients

  - Length of the cavities is about just shy of 7 m. Some warm-to-cold transitions.

  - This assumes permanent magnets again.

- Vasiliy had trouble fitting all the energies into the linac from the FFA arc

- If the beta values at the end are always the same, easier to match.

  - Blue is betax_max

- Vasiliy: lowering the size of the betas would be beneficial.

- Dejan: don’t care about middle of linac that much.

- Combine both triplets and final focus quads in the linacs to bring down the betas.

  - The absolute value matters less than the spread of the betas for each energy at the end.

- What happens to spreaders?

  - With single FFA, come out of BCOM, then it’ll replace the current EM 5th pass.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 minutes | Agenda topic AOB | Presenter All

- Scott – one mistake we’re making is being matched into LINAC. If you didn’t have any focusing magnets, then you have a DoF available to you, what the beta-star is in middle (and that determines on the end). If you insist on being matched at a high energy to the triplets or FODO, you’ve lost DoF. At a very high E, you must be close. Effectively, you could give yourself that DoF, essentially treating the chain of magnets as a moderately perturbed drift.

  - Alex – doing this with ERL work

  - Can play with optics so that we have a “ratty looking parabola” in the higher energies, and control in the lower.

  - Still control beta at ends.

  - Might blow up betas to large numbers

    - Looked at this a bit, but abandoned before. Maybe re-investigate.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g