# FFA@CEBAF Working Group|Minutes

## Meeting date | time 7/22/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex C. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Alex C, Stephen, Kitty, Andrei, Kirsten, Dejan, Randy, Vasiliy, Reza, Amy |
| --- | --- |

# Intro Discussion

Ryan gave LDRD “Defense” talk for the renewal. Went well.

# Agenda topics

## Time allotted | 25 mins | Agenda topic Weekly Focusing Linac | Presenter Alex B

- New alternative optics called “weakly focusing’

  - Drift linac at lower E

  - Beta is parabola in x and y – same values of beta and alpha feeding into the FFA arcs

    - Differs by ~4 mm

  - Don’t want 200 m of beta

  - Last week, showed 18 degrees per cell, now 30 degrees per cell (optimum)

  - 1st pass on top

    - All cells matched with flat profile

  - Beta and alpha seem decently controlled

    - 45 m difference in beta in WF linac

  - Can these values be used with Vasiliy for matching?

  - Values at end of LINAC

    - Has to go through spreader and splitters

- Can put small quads to match?

- We have to match down from 220 m to 20 m for Vasiliy

- 5th pass splitter will be for all 6 beams.

- 4 or 5 passes in one FFA is reasonable. 6 Might be possible, but unsure. Might be too crowded for splitters

- Both linac options are similar for optics at the end.

- Dejan: splitters – design magnets with open plane

  - Can create dipoles with more than 2 T field

- Stephen: maybe use a gradient magnet to separate out the passes further.

- Focus on 1 FFA first, then if we need more we can re-adjust.

- Vasiliy: So we need the spreader and splitter in between the arc matching and linac?

  - Yes.

- Dejan: forget about CBETA electromagnets in splitter – we’ll need to have open-aperture magnets with high field.

  - Stephen: we’ll need some EM for tuning

  - In small place with strong magnets, could use Sumerian Cobalt magnets (less susceptible to radiation damage)

    - All losses at CBETA were in the splitters, not FFA.

| Action items | Person responsible | Deadline |
| --- | --- | --- |
| Make Spreader 9+ | Ryan |  |
| Make splitter |  |  |

## Time allotted | 25 mins | Agenda topic Adiabatic Arc | Presenter Vasiliy

- To proceed to matching, need to know boundary conditions.

- We have arcs to large beta matching solution

- With spread out beams, the beta functions could be brought down

- Should focus on reducing length of matching section by reducing beta

- From linacs, we have a 45 m spread in the beta – perhaps we can mitigate this?

- The solution we have now is for the values presented above

  - We can assume the spread of the betas is reduced and see if it makes matching easier.

- The hard part for Vasiliy is the existence of the spread of betas.

  - Easiest way is to use quads in each energy line to match betas down from the linac values.

  - Try to match down to Vasiliy’s values.

- Area of splitters with free space, put quads for matching

  - 6 separated beamlines for 6 energies in single FFA

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

- Stephen shows his magnet prototypes!

  - 1.68ish T inside good field region

  - 0.4 T on the edges

- Ordered more stuff to make longer ones.

- When you quote the area, is that the sum of the wedges?

  - Yes.

- General discussion on how assemble/stick together/measure, etc…

|  | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g