# FFA@CEBAF Working Group|Minutes

## Meeting date | time 7/29/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex C. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Alex C, Scott, Reza, Randy, Vasiliy, Dejan, Amy, Stephen, Kirsten, Todd, Jay, Kitty |
| --- | --- |

# Intro Discussion

Alex Coxe will provide notes while Ryan is speaking. Thanks!

# Agenda topics

## Time allotted | 25 mins | Agenda topic NE Spreader Redesign | Presenter Ryan

- 10 pass spreader Redesign

  - Uses Alex B’s linac optics

  - Focus on closing dispersion

  - Extra quad near the end of linac files, so optics may change a bit

  - Ryan will fine tune later as needed

- Alex says 1.3 T is probably the max strength

- Jay says no show-stopping magnet problems

  - Avoid shunts and whatnot

  - Seems to be reasonable now

  - We will need new magnets

- FFA pass line might need a septum magnet which could be tight

- Values are different between spreadsheet/Optim, small quads close dispersion

  - Possibly due to fringe fields or rounding errors

- Stephen and Alex want to know beam offsets

  - Ryan doesn’t have them yet, because each line is separate in OptiM and the view orbit function only shows the orbit for that specific energy, which is, by design, the height of the beamline

    - Ryan is unsure of how to get this in OptiM

  - Ryan mainly focused on closing dispersion

- Everything is great with the quads on

  - Seems to be preferred by some so have control over optics

- Jay says might be getting SR losses due to high energy

  - Dispersion differences between energy levels are not small

  - Dejan thinks it’s about the codes

- Jay says don’t mix vertical and horizontal dispersion correction

  - Scott and Dejan concur

- About 20m left for horizontal splitters

  - Might want to shrink the spreader

  - EM hardware might get in the way

- Might want to shrink FFA pass

  - Longer magnet

    - Would need conductive cooling

  - Drift is fully adjustable

    - Can be longer or shorter as needed and won’t impact the optics/dispersion

- We haven’t looked into extraction yet

- Hall D can be extracted at a lower E

- Alex wants to see vertical separation

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic FFA Arc Errors | Presenter Alex C

- Redid error studies for Dejan’s lattice – slightly altered to be more like FODO cells

  - Sum comes very close to Dejan’s lattice

- Duped Dejan’s reference energy.

  - Kirsten – let’s talk about energy offline

  - Got more of the same.

  - Losing just under 1 GeV without correction (also the whole beam)

- Error bars only show w/errors, since errors in the ideal case are all ~3 mm.

- Different than last set, where consistently off by a set amount.

- Will implement with correctors and BPMs later.

- Need to look at where orbits start to blow up in general – usually pretty stable with small oscillations for 10-20 m before they blow up.

  - Will iterate piecewise

- Dejan: don’t like the first picture. Sampling point problem. Doesn’t look like you’re starting with the correct lattice.

  - Not enough points plotted. If you plot all the points, it’ll look better.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

- Costing exercise.

- Use LERF infrastructure.

- Cryo load concerns.

- Steve Benson gave input

  - Need c-100s but will surpass cryoload

  - Other options, playing with other cryomodules.

- Jay: won’t fit in 10 m. Assume it’s 20 m wide. LERF needs to be 20-25 m wide and 75 m long.

  - No space to put injector in top left.

|  | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g

NO MEETING NEXT WEEK!