# FFA@CEBAF Working Group|Minutes

## Meeting date | time 8/26/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex C. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Jay, Vasiliy, Kitty, Kirsten, Alex C, Andrei, Stephen, Dejan, |
| --- | --- |

# Intro Discussion

JLab all staff scientist meeting this afternoon. Later, 3 town meetings all about our upgrade. NSAC got the charge to do this in July. User’s meeting on Sept 8th.

# Agenda topics

## Time allotted | 25 mins | Agenda topic Panofsky Correctors | Presenter Jay

  - Italics is from Stephen

    - Based on gradients and alignment errors, need ~150 Gauss for dipoles (both planes)

    - Used all headroom in CBETA

    - Would want 2-3 x 60 mT/m

  - Magnet is pretty much “all ends”

  - Since permanent mags are longer, we need a provision for aligning permanent magnets

    - Assuming happens at each end

    - 20 cm square (b/c quad)

    - Allows support and alignment

      - May be +/- in size a little

  - Could get a 250 W or more from quad, close to 500 W from each dipole coil

  - Getting to point where you want some cooling (even if conductive)

  - Need larger conductor

  - 14 kG at steel at max excitation

    - Reducing steel thickness isn’t an option

  - Chose 1 inch b/c we have it sitting in boneyard, so we could prototype

  - All ends

  - Can get up to 400 Gauss with this

  - Can maybe do air-cooling, but there’s a gap there, so we can fit 1/8th inch copper for pipes with conductive cooling

  - 35C is the water temp for cooling – impact?

    - Dejan: yes, maybe.

    - Stephen: CBETA was about the same – might make the magnet a percent or so weaker.

      - Have some channels in the framing of the permanent magnets themselves as well as the cooling outside

    - Stephen: might be smart to get a chill plate separately and clamp it on.

- Space left for water cooling on inside and outside in the 1/8th inch gap (corners of box)

  - Can also do it adjacent to aluminum block

  - A ladle is 60-100 tons depending on vendor

  - Should be able to buy the chemistry we need, but should do it all at once to make sure we get BH curve needed for the magnets.

- Stephen: length? Roughly equal to length of permanent magnets, maybe a little shorter.

  - Chose 40 cm – typical alignment blocks are 4” squares

  - Assumed 60 cm permanent magnets

  - Depends on lattice – as long as *most* of the length has the corrector on it should be ok

  - If you make it longer, you just get the additional strength

  - Cooling needed if make it longer?

- Dejan: This is much higher energy

  - Very nice job – exactly like BNL lab did

  - Jay: just wanted something basic so engineers on costing had an idea

- If they’re a meter long, get 2.5 x integral shown

- You don’t want to go thinner in the steel – this is pushing the quads and 1 dipole at max (why it’s on two sides).

- Depends on how well we can align magnets and the overall quality.

- If it ends up only 1 T, you’re in linear regime and no issue

- Stephen: looks a lot like CBETA, and always want more headroom rather than less

- Alex C: Part of the MC correction scheme includes an optimal position parameter

  - Don’t know how long they’ll have to be – please keep me apprised, since my code accounts for where

  - Jay took 10 cm off each side of full magnet length thinking about alignment

    - Rounded assuming 60 cm dipole.

    - Made this 40 cm with steel, but more like 50 cm with coils

    - Subtract 15 cm from each end of each permanent magnet, and they’re in the middle

      - And *MUST* be in the middle

- Alex B: one corrector for each permanent magnet is assumed

  - Will depend on the number of cells to know total number

- At least make sure buildings have enough room for power supplies for each of these.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic Splitter Requirements | Presenter Alex/Scott

- Splitters will be most involved new system. Making requirements document

- More coming, and will update as the design is updated.

- Vasiliy – can you provide what we’ll be matching into?

- Dejan – could you please share the required path length and M56?

- Draft will be posted in presentations folder.

- TOF splitters give a lot of flexibility – at top E when exiting, we can independently correct path length and put in some momentum compaction

  - This may help address some of Jay’s concerns.

  - At 6-sigma, troublesome

  - We can do the same thing we do at CEBAF, and do longitudinal compression, trading bunch length for momentum spread

    - Rotate by quarter wave and help momentum spread.

    - Jay: Has to apply to EM passes as well

      - Alex: it’s mostly created by last two passes

      - Must be fully compatible with parity experiments as well

- We could add change log

- Numbers will be added/updated as we go

- We can add in dimensional limitations (longitudinal and transverse)

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

  - In meters, for 98 cells in one arc

  - TOF is usually t_part – t_ref, so this looks opposite than normal

  - Already TOF in first section was of this order from vertical spreaders

  - Spreaders give small negative M56 component

- Dejan will update and refine

- Andrei/Dejan spoke – good discussion and path toward having this mentioned in the NSAC

  - Getting reviewer comment removed (?)

- Andrei: there are plans – the first meeting is today internally

  - Next week, users meeting

  - JLab participate with town hall meetings arranged within long range plan

|  | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g

NO MEETING NEXT WEEK DUE TO LABOR DAY