# FFA@CEBAF Working Group|Minutes

## Meeting date | time 9/9/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan, Alex C. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Jay, Alex C, Stephen, Dejan, Randy, Kitty, Kirsten, Scott, Todd, Vasiliy |
| --- | --- |

# Intro Discussion

Thanks to Alex Coxe for taking notes while Ryan is speaking. They are added in below.

# Agenda topics

## Time allotted | 25 mins | Agenda topic Spreaders > BMAD | Presenter Ryan

  - Glitches?

  - From patches, and magnet representations not accurate in transverse dimension

- Ryan working with Kelly on 3D model

  - Can it be done in BMAD?

    - Yes, but not 3D

    - Already working with Kelly

      - Some sort of permissions problem

- Design needs to be refined.

  - First pass looks good, optically, comparing BMAD and OptiM:

- Used overlay to make calculations more convenient

- Direct conversion from OptiM to BMAD complicated

- Quick backup: we are discussing the 1st pass vertical spreader

- Quick aside from Alex:

    - OptiM uses path length rather than steel length.

- Scott: key point on doing it in BMAD – for every pass, the magnets are the same magnets. When you do the OptiM files, you have to do calculation for every single pass.

  - This doesn’t happen in BMAD. You just lay them out.

  - All other passes go through common magnet.

  - The code does it right.

  - Now let’s put some fancy ends, or add in field maps, you just do it.

  - Purpose of patches is so that you can align coordinate systems with different patches

    - Snap it back to align with that line

    - Also tells you where the particles exit the magnet, and the angle they are exiting.

- Question about incoming beta – too large?

  - Based on weakly-focusing linac, first pass, so smaller

  - Negative M56?

    - Yes, and positive in arc. Spreader + ARC + Recombiner cancel out M56

    - EM arcs have tunability, so M56 not such a problem.

- Alex B: next steps?

  - Ryan: Keep iterating on Pass 1 with different methods

    - Then go through all EM passes, then FFA passes

    - Finding “right” method

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic Adiabatic Arcs | Presenter Vasiliy/Randy

- Randy will present

- 650 MeV injector, 1100 MeV/linac

- 4 x EM passes, then 5-6 FFA passes

- Will upload the table to the shared repository

- Values are “wish” values, they’ll be the same as in ARC FODO cells minus dispersion and orbit offset requirements.

  - Can still increase beta functions using Dejan’s scheme

    - Phase advance must stay constant

    - Can probably increase by 2-3x

- Can iterate as needed

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

- Stephen:

  - Similar to our FFA magnets

  - Aperture is most difficult part to get

  - Can maybe get 8 mm if cut off corners

  - 20 mm vertical aperture was very expensive, so shrank it down for affordability

  - This has plastic plug to hold things in place

  - Sign change at green part on right (negative sign on some of fringe field)

  - Orange and blue are field error

  - Brass shims helps the field from wedges falling in

  - Iterations based on revised measurements (No Tuning Rods on plot)

    - Kept improving

    - All errors below 10^-3

  - Short magnets, you don’t get to full field, and tuning isn’t so strong

  - Red is integrated By field (B/L)

  - Ignore last 2-3 points – they are outside good field region

  - Scale different

- Probably more accurate if epoxied wedges, but didn’t b/c don’t have many and need to reuse

- Would be better with metal, etc…

- Temp variation and thermal coeff come into play as well.

- Scott: in later iterations (slide 15), looking at errors, they look non-polynomial. Odd kinks in behavior

  - Is your method to decide on shim placement based on polynomial?

    - Just put in measured field, then optimized

    - Point-by-point

- Jay: Senis (sp?) probe has accuracy issues at this level.

  - Stephen:

    - 10^-4 is level of accuracy

    - Probably see positioning errors as well

    - Enough to get magnet to the level we typically need

- By hand?

  - Some plastic cartridges, but yes. Can get to ~0.5 mm by eye.

  - Jay: make sure starting from corrected field each time

    - Had a patent application (later stolen at GE) for work like this

    - Objective function changes by iteration

    - Can sent patents in name and ones swiped by others.

  - Stephen shimming in 1D (integrated)

  - Jay/GE was doing 3D shimming

- Slight issue with this method: takes up space in aperture

  - Has ideas to improve (wires between tape, for example)

|  | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g