# FFA@CEBAF Working Group|Minutes

## Meeting date | time 11/04/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Jay. ### Note taker; Ryan/Jay. ### Timekeeper; Jay. | Attendees Ryan, Jay, Kitty, Kirsten, Donish, Scott, Stephen, Dejan, Vasiliy, |
| --- | --- |

# Intro Discussion

BMAD on ACE – proxy changed? Problems started roughly last week.

Thanks to Jay for helping with notes while Ryan presented.

# Agenda topics

## Time allotted | 25 mins | Agenda topic 6-Pass Magnets | Presenter Stephen

- Lots uploaded to the SharePoint

- Wanted magnets that are compatible with Dejan’s lattice in some way

- Got magnets that look ok.

- West arc magnets are a bit larger since higher energy

- 16 mm for total height

- 12 cm wide

- Only got these with some difficulties

- Field errors:

    - Grey is error

  - These are field orbits – plots of actual field in 6 orbits.

    - Top plot is original design

    - BD has the inverted parabolas, BF standard parabolas

  - What happens if magnet follows orbit to some degree? Can follow any orbit

    - Try to make magnet follow orbit of highest field – basically cut off the peak

  - Bottom plot splits magnets into 6 sections, shift sections mm scale

    - Peaks essentially removed, looking at average instead

  - Peak fields higher than expected

  - 2 ways look at max field. One looks at max orbit, one looks at orbit range

  - Getting larger numbers than expected (in BD for example)

- Looking at parameters spreadsheet, looked at min and max

  - Hit max field while exiting

  - If you treat it as a hard edge magnet, you get to the max value.

- New magnets drop from peak of 1.7 T to less than 1.6 T

- Scaled magnet gradient to 0.6 of what they were, so dimensions adjusted a bit

  - Longer cell length and orbit excursion

- Magnets less than 100 cm^2

- Used real energies (with SR loss), instead of nominal

- Still want to widen the open plane gap, if possible

- Muon1 folder shows elements split into 6 pieces. Angles are now 0, and there are half corners (drift with angle and no length).

  - Has input files and listing of components so can see what is going into simulations/design

  - Opticsmatch spreadsheets go into more analysis, comparing options, etc…

  - Also has outputs for beam tracking

- Files in Presentations>2022-Nov4>muon

- Higher gradient gives shorter cells by sqrt relation (98->76 cells for example)

- Dejan used sector bends in his version

- May have improved SR loss – but TBD. Likely very similar.

- Dejan wants to show the resolution of the M56

  - Number of cells in arc and calculate parameters of cell (dispersion, offsets), you get M56 at end of file.

    - Corresponds directly to the momentum compaction for that energy divided by circumference in first column in MADX-PTC.

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic Spreaders | Presenter Ryan

- Showed spreadsheet with version 3 of NE Spreader and version 2 of SW Spreader.

  - NE V3

  - Grey is EM passes, Yellow is FFA passes

  - SW V2

  - Grey and Yellow same. Green shows two options for pass 8 (raising arcs vs stronger quads)

- Mostly been working on NL into SL.  SL Version 3 is clearly better for intermediate passes.

- Negative drifts will have to be removed at some point

  - In both NL and SL, but more in SL

- May be able to tune splitters to improve beta values match at end of each linac

  - Unsure if possible, or necessary

  - If can change input Twiss for each pass, can make all match at end of linac

- Need beam match data exiting vertical spreader and entering FFA to design splitters

  - Likely to change as we iterate

  - Now have a version of output from spreaders, and a version around the FFA line, so can determine what the splitters will match from and to.

- Try to get to H separation quickly rather than having a triplet before splitter.  (Jay- that triplet will have to have large aperture, so long)

|  | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

- Scott to Kirsten: still having SR problems? Some bugs fixed.

  - Problem about gaining energy got fixed

- Another problem related to quads – doing radiation wrong

  - Kirsten hasn’t noticed this problem from October version.

- Kirsten will run through 3 different versions of BMAD and report to David Sagan.

  - Comparing package manager and tarball versions with different Linux versions

- Work in progress…

- Package manager version seems OK for now. Just maybe don’t use radiation.

|  | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g