# FFA@CEBAF Working Group|Minutes

## Meeting date | time 11/18/2022 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Alex C, Vasiliy, Dejan, Jay, Todd, Donish, Reza, Scott, Stephen, Kirsten, Randi, Kitty, Andrei |
| --- | --- |

# Intro Discussion

IPAC: Alex B will draft a whole collab submission. Deadline is December 7th.

https://www.ipac23.org/dates-and-deadlines/

Think about other IPAC submissions as well. Stephen put in 2 (1 for magnet design).

Reza - Injector part? Alex B – will be mentioned in general paper, but we could also add an abstract for that. Reza is sending one for positron part.

# Agenda topics

## Time allotted | 25 mins | Agenda topic M56/Path Length FFA Arcs | Presenter Dejan

  - Alpha is momentum compaction here, not Twiss

  - T/To = how control longitudinal motion

  - Directly connected to splitter design and M56

  - Everything positive

  - Negative dispersion in central dipoles (blue)

  - Are you getting negative or positive value? Track this to see what you’re doing.

  - From June-ish

  - Almost all is positive dispersion – the integral divided by rho will generally be positive

  - Sometimes, you get parts that go negative – then the integral can become negative

    - This would change the splitter design

  - Didn’t take into account SR loss

  - M56 in meters here

  - Took whole lattice, put it in MADX-PTC, SYNCH, and BMAD

    - Pretty consistent between codes

  - MADX-PTC – it is negative due to definition vs. SYNCH code. (- delta-s)

    - Same thing happens in OptiM – likely due to a convention.

- You want these numbers correct in the splitters exactly (with opposite sign)

- Alex B – there will also be a small negative coming from the spreaders – so we need to make sure we take that into account as well.

  - The table is only the FFA contribution

  - Need to be sure that this TOF is corrected as well.

  - Be careful with sign

  - This is momentum compaction times circumference

- Jay – TOF is also important

  - 12 GeV moved magnets an integral number of wavelengths

- When design splitters, you have to fix the geometry for TOF first, and choose which bucket you’ll deal with.

  - Then M56 changes phases of quads

- Can juggle positive and negatives (zigzag)

  - First, try cancel TOF, then adjust phases by zigzag or other “bumps” to get dispersion integral correct.

- When you do the M56 calculation in BMAD, you have to do the whole lattice.

  - Fairly straightforward

| Action items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic Adiabatic Arc | Presenter Vasiliy

- It’s no longer adiabatic! Simply not enough space, so use quick non-adiabatic match using just a few magnets.

  - Extremely convenient to look at different energy optics

  - This is arc cell with multiple plasses

  - Min design is two s-bends

  - Two arc cells connected together, turned off dipole field in all focusing magnets

  - Flipped the polarity of the second

  - Optimized using the length of the magnets and drifts in between

  - Design and error dipole fields

    - Design controls the design orbit, error controls how far away from design

    - Varying both at the same time you can move things together

  - Design bending angle is changing if you vary them together

  - Varied gradients, etc…

  - May need nonlinear fields

    - Dispersion not linear function

- Can do it with linear fields for two selected orbits, but then the rest don’t work.

- Used more parameters than needed, so will look at getting rid of some.

- Lowest energy always the biggest problem – Dejan recommends adding weight to that.

  - Takes time to get the weights right, as well as step sizes.

- Orbits merge beautifully.

- Alex B – this is very compact! 10 m in s-coordinates

- Dejan – the first magnet has to be a quadrupole because it impacts the orbits differently for each energy.

  - Larger the offset would make a kick toward the middle

  - 1983 article presented previously.

- Scott – skip pole face angles – probably equivalent to doing something in the body, and it’s likely difficult to implement in reality.

  - Probably accomplishing a lot through nonlinearities.

- Stephen – you need dipole, nonlinear + nonlinear to match orbits. If you want Betas too, you’ll need more magnets.

- Vasiliy – getting the extra focusing terms from edge angles.

- Dejan only used gradient and bending in his fitting. Start where orbits are separated, and that magnet has to be a quad with very little dipole component.

- Advantage of BMAD – no other codes can fit multiple universes simultaneously.

  - Mad-X can do it, sort of. But harder and slower.

- Scott: should scan mismatch parameter as well at end.

  - Dejan’s solution – 4-ish magnets

  - Orbits merged, one beta is going nuts

  - Slopes of Beta at end is 0

  - Dispersion is matched.

  - All linear magnets – only distance, gradient, and bending are adjusted.

|  | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

- Kirsten – SR radiation loss issue has been sorted out. More confident that next iteration of emittance budget will be more accurate.

- Stephen – uploaded something ~a month ago for comparison.

  - Made fake sector magnets similar to Dejan’s, then worked things out.

- Split rectangular magnet, shift it a bit so the beam doesn’t wander into a stronger field region.

  - Alex C has this for one arc

  - Stephen will check to see if offsets for other arc are there.

|  | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g

NO MEETING NEXT WEEK – HAPPY THANKSGIVING!