# FFA@CEBAF Working Group|Minutes

## Meeting date | time 10/20/2023 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan, Donish. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Alex C, Dejan, Donish, Edy, Scott, Kirsten, Randika, Reza, Stephen, Todd, Vasiliy, Nick, Roger |
| --- | --- |

# Intro Discussion

- Ryan (notetaker) late (11:04) so Donish took notes as well

# Agenda topics

## Time allotted | 25 mins | Agenda topic Multipass Correction | Presenter Alex Coxe

- Work focused on west arc

- Based on how error correction was done in CBETA (via Kirsten's guidance)

  - No quadrupoles

  - SVD correction (from lowest to highest energies)

  - 1 more bpm than correctors

  - 5 configurations shown here.

  - Tests with 150 correctors in arc (1 on each magnet)

    - Very effective

  - 100 correctors in arc

  - This is a view of 1 cell (BD + BF)

  - For each arrangement, there are either 1 or 2 corrector setups. Moving correctors longitudinally on the magnets

- Scott – how long are the magnets? – 1.3 m for BD

- Correctors are implemented as controllable elements in Bmad, but they are 40 cm long, based on Jay’s Panofsky design

  - Integrated kick over 40 cm

- Currently working with 100 correctors in the whole arc

  - First 10 cells have 2, last 15 cells have 2, then the in-between has 1 corrector each

- Scott: why did you focus on leaving it on the first, upstream magnet? Tradition would be to alternate them

  - Interior magnet configurations (not on the front edge or trailing edge)

  - Upstream does order of magnitude better

  - Downstream does best

- Scott – betas peaking in middle of each magnet

  - Curious that there’s a preference for trailing edge

  - If you have symmetric betas here and every drift is the same, so why is trailing edge better?

    - Artifact of measurement?

    - If you look at an invariant measure, I would expect leading and trailing to be similar

  - Moving BPMs either on upstream, middle, or downstream edges of drifts

  - Small differences

  - Initially, BPMs like that on left (at beginning of drifts) – same number of BPMs as correctors in ARC except at exit (number of correctors +1)

  - Tested all three of these:

  - Any configuration is OK, but lowest merit is at the end of the drift

  - -1 mm to +1 mm, so up to ~10 microns

- Scott: when you talk about merit you’re looking at BPM positions over a long section

  - Yes – this is for the full arc

- Todd: doing correction with SVD (tailored)

  - Get list of singular values, are you doing a SV cutoff, and if so, how many SVs are you cutting off?

  - Alex C – it’s built into Bmad, set cutoff at E^-6, so it does it automatically.

    - Haven’t looked at response matrix

  - Todd – if you have a ton of redundancy, you can throw away more BPMs/correctors

    - Kirsten – KEEP THE CORRECTORS!

  - Stephen – might be useful to move the cutoff around a bit and see the response, but suspect it’s not cutting too much

- Dejan – two different response functions

  - Stephen – no, only one. One is correctors, the other is BPMs

- Kirsten – when you say set of errors, is that just directional misalignment, or rotations, etc?

  - Right now, just offsets. Need to run more on the farm

- These include position modeled at other places.

- The final BPM/Corrector situation is

  - Downstream BPMs, trailing edge correctors

- Alex: lattices doing this on, just have orbit and momentum in X direction

  - The ones with correctors are on GitHub and Sharepoint

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic Transition Lattice | Presenter Vasiliy/Randy

- Last night, thought could show something. But turns out got stuck at pass 5

- Tune separation that thought would work not large enough

- Vasiliy – could meet all constraints except one, but trying to meet that leads to a large beta-function blowup

  - Basically, same harmonic getting in resonance with two passes, so you must carefully chose betatron per cell so it’s only for a single pass

  - Avoid harmonics of other passes

- May have found new set of tunes, but need to run it

- Compare each pass with another, calculate different tunes – anything close to 1 isn’t good

- Need to run and see if it’ll work

- Kirsten – is 100 m the length?

  - No, it’ll be shorter

  - Old trials without everything in it were 16 m, but it’ll need to be much longer

- Todd – some of this is similar to stuff Dave Douglas did, might be worth reaching out or finding papers

- Kirsten – this includes the Recombiner?

  - Ryan – it’s the chicane without quads, about 30 m drift

- Kirsten – do we have spreaders for the new optics?

  - Ryan, everything was based on the weakly-focused linacs, and so we’ll need to go through and rematch everything for the new optics as they are available

- Alex B: Donish is automating this

  - Donish: nearly-automating. Work in progress

- Kirsten – may look at radiation integrals in spreaders/recombiners for strongly focusing linac optics

- Alex B – good numbers for longitudinal emittance

  - Kirsten – maybe I should recheck my latest numbers

- Vasiliy:

  - Adiabatic match (this isn’t CEBAF, but similar)

  - With finite number of matching cells

  - Dispersion not fully suppressed, starts ringing – we’ll likely need to consider this (dispersion leakage after adiabatic match)

  - Need a knob to control dispersion and orbit, preferably by individual passes 1 by 1

  - Same lattice, but correcting dispersion

- Dejan – In CBETA, 4 passes, had to avoid 0.333 resonance if tune is in cells close to 3rd order, then problem

  - Avoided going through 3rd and a few other resonances, picking up tunes for each energy to avoid it

  - Didn’t do it for 6 passes – all will have to be away from 3rd, and cells are numerous

  - Go back to lattice design to avoid those

  - Depends on max/min tune so you can choose 6 not to hit the third

- Scott: be careful, need energy variability

  - Cryomodule loss and/or user preference

  - CBETA had large energy range

  - High phase advance per cell is the hard spot, jumped over that in CBETA

    - Not hard to jump the hard part at CBETA

  - Here, problem is that the relative energy changes are small, so dodging things is harder

    - Given that we want to dial things, this will be even harder

  - Probably should take lowest and second lowest energies and have them straddle the 1/3rd

    - Or representative energy

    - See what happens when you vary the range you want to cover

  - Also need to worry about ¼

  - Can probably live with all else

- Dejan – if you fix/reduce the tune changes, you get into a problem of how to correct orbits for different energies

  - Maybe add sextupoles? Reduce variations. Unsure. Need to study more carefully

- Kirsten – is there an example of one of these transitions that can test for radiation integrals? Just to look at scale?

  - Ryan – I suspect “not ready for primetime yet”

  - Vasiliy – just the section with dispersion?

    - Yes, if it’s not bending, doesn’t matter

- Vasiliy – to make it useful, need to combine dispersion suppression and beta excitation sections.  We can work on that. This was the next step

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g