# FFA@CEBAF Working Group|Minutes

## Meeting date | time 04/19/2024 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Alex C, Donish, Edith, Kirsten, Randika, Timur, Dejan, Todd, Andrei, Reza, Stephen, Vasiliy, Scott, Tim, Sushil Sharma |
| --- | --- |

# Intro Discussion

- Dejan happy with his work – apparently will be part of LDRD

  - Salim may help

- FODO arc is a “boring lattice” – if we can massage dispersion and make 25 supercells, might be more interesting

  - Dejan – DOE likes FODO

  - AlexB – we can call it “FODO-like”

- Good to have alternatives in pre-conceptual design

  - Will put extension of perturbed FODO into concept paper

- Dejan – trying to see if other codes show same thing. Translating to 3 other codes.

- Alex Coxe update – responding to review comment: 6th pass not so limited. Aperture limit imposed that was incorrect

- Ryan mentions uploading things to IPAC folder

# Agenda topics

## Time allotted | 25 mins | Agenda topic NSLS-II Magnets| Presenter Timur Shaftan

- Timur Shaftan joins with mechanical group leader

- Exciting designes for permanent magnets for light sources – started at Grenoble, continued by BNL

  - Replace magnet assemblies with complex bend lattice

  - 6 combined function focusing PMQs

  - Replace 1 dipole and collection of quads

  - Worked with Stephen Brooks on this

  - Compact

  - ~130 T/m, 16 mm aperture

  - Reduce energy consumption by over 80%

  - Iron-core dominated hybrid option under fabrication

    - Good control over field quality

    - Space for BPMs – vacuum chamber not so tightly integrated with PMs

- Pursuing 2 designs – Hallbach and Hybrid

- Concerns on PM radiation hardness – looking into this

  - 10 kW xray in broad spectrum

  - Measuring the PMQs

  - Rotating coil good for measuring small aperture magnets

  - Plan to measure – before and after install

- Running 29 beamlines with IDs – 1-3 m long undulators (4.7 mm gap)

  - Extract 8-10 kW

  - Sensitive detectors

  - No degradation of spectrum in beamlines

- Recently hired PhD engineer from DESY – good with radia program. Day-scale turnover

- Alex B – can you define the radiation environment?

  - Two ways to adjust: magic fingers w/ small PMs

    - Different combinations can adjust harmonics

  - Numerical simulations can adjust ~50 units of harmonics

  - Another method tried and works – EDM the internal aperture of Halbach quads

    - Adjust up to 200 units by profiling internal apertures of Halbachs

  - For hybrids, allowed enough room to adjust harmonics

- Scott – burning question: EDMing permanent magnets. Thought blocks come coated. Does that mean you’re EDMing the coating?

  - The coating is required for NeFeB, but we are working with SmCo

  - Problems cutting through parts – had to try a few times

  - Does EDM do heating?

    - Yes, it demagnetizes about 10-15 microns – taken into acct during optimization

    - If we stick with this design, EDM is not production-friendly. Will eventually move the gradient

    - Modifying internal aperture adjusts harmonics. But in production, we’ll do this differently by moving them in and out

- Dejan – isn’t that hard?

  - Just do a few, then automate

  - What is reason for large error in harmonics when assemble magnets?

    - 1 – magnetization angles if you want to go with the usual range could be off by 2%

    - Got better as well, but 1% level is still large at times

  - Maybe assembly is the problem?

    - Can be – internal aperture could be off by 100-200 microns

  - Made from spare parts

- Ryan – do you directly measure dose?

  - Yes – RadCon can

  - Also use PMQs – effect from electron beam

  - Multi-bend-achromat, but adjusted

  - More space for elements in arc

  - Emittance is near diffraction limit

  - Complex bends are 1.5 m each

  - Flips gradients in PMQs

- Doing error simulations now

- Presented to basic sciences committee – they say approach is excellent, they are hoping for significant funds to finish both Halbach and Hybrid configurations

  - Will place in tunnel

- Dejan – did you get picometer emittance?

  - At moment, 23 pm-mrad

  - If go round beam, and insertion devices helping

- Dejan – would you replace all magnets with PMs?

  - Not all, sextupoles have too high requirements

  - Stephen – you can impose sextupoles on combined function mangets, but then you can’t change the sextupoles

  - Combine sextupole corrections around Halbachs – allows superposition – likely in the future

  - Timur – not for us

- Scott – wondering for Halbachs, why not choose wires in aperture method of correction used by Stephen. For correcting errors in Halbachs, wires in aperture can be used for this.

  - If you want to achieve very high gradient, there’s no room. The vacuum chamber is right against (0.5mm gap) magnets. There’s not structure to accommodate the wires – the wires themselves are 1-2 mm themselves.

  - Stephen – like the wire EDM system, but you have to get it right the first time. But it’s great for small aperture magnets.

- Will make new branches for further testing of magnets

- Ryan – SmCo is very brittle – how are you dealing with it?

  - They had that issue too

  - Chippng didn’t cause much change in harmonics as assembly errors

  - Now have in-house assembly method with no chipping

- Dejan – KYMA work, and brass spacers to use epoxy

- Scott – our magnets were coated, right?

- Sushil – suggested coating to reduce chipping, but it didn’t help.

  - Halbach on left, hybrid on right

- Dejan – Scott found that 12 magnets in a row had crosstalk between iron pieces, so went with Halbach

  - Scott – found that this was manageable. Able to do the calculations for crosstalk. When close together, flux from iron in close magnets is real

  - Sensitivity to worry about, but no worse than error analysis done for placement of Halbachs, except more involved – with Halbachs, just superimpose. With iron, do the full simulation with multiple magnets to see errors in each.

- Stephen – crosstalk made us lose strength with Iron magnets. F magnet lost strength to D magnet.

  - Scott – my recollection is that Dejan thought they were too big

- Dejan – took 1 year to convince 3 comittees to go with Halbach

- Stephen – reason magnets were big b/c they had to be made bigger to deal with crosstalk

- Timur – we already discussed. Don’t compare FFA to light source

- Reza – quick question: those “magic fingers” are half a circle? The number of wedges you have, were they the same number of wedges you have for PMs, or you want to keep multipoles corrected?

  - Wedges is determined by mechanical issues. More fingers you have, the better. 8 wedges in Halbach, have 16 or more in magic fingers.

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic LDRD | Presenter Donish

- How expand scope? Package differently?

  - Ryan – be careful pitching 22 GeV instead of energy upgrade. 12 GeV was a mistake.

  - Alex B – but we already pushed it, so we’ll stay there.

- To address Scott’s concerns – including splitters and transition. Dejan’s work will change requirements on splitters, or remove the need of them

- Ryan – be careful how you pitch the S2E – rules state it can’t be a continuation of a previous LDRD/study

- Scott – didn’t want an expansion of scope. Wanted to redirect scope instead. Need S2E, but understand that have to be careful how you pitch it.

  - Want to steer so that more in direction of accomplishing S2E.

  - First bullet – delete –

    - When I go to PAC, and see “generative AI” I just stop.

  - Leverage the funding to get to the point where we have a tracked design to present

- Alex B – can change last bullet. It’s about different changes to overall design.

- Ryan – Scott – are you saying we’re splitting focus?

  - Scott – no, we’re trying to go from 0 to 1. Not 1 to 2 or 0 to 2.

- Dejan – Scott is right. We don’t have a complete solution. That’s the starting point. Need to finish connection of pieces. Then we explore other pieces.

  - Alex C agrees.

- Alex B – so maybe truncate study. Not at level where we’ll be improving things, but at the level on how other systems can take some functionality to make it easier to achieve splitters

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g