# FFA@CEBAF Working Group|Minutes

## Meeting date | time 06/07/2024 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Donish, Dejan, Edy, Reza, Salim, Scott, Kirsten, Stephen, Randika, Vasiliy, Roger, Todd |
| --- | --- |

# Intro Discussion

- New email list for physics case(s) for energy upgrade

  - Alex B is liaison

  - Meetings every-other-Monday to discuss

- ERL in Japan this year – tell Scott about good speakers.

  - Maybe Kirsten – Scott has the POWER!

- How far apart are ERL and FFA?

  - Not back to back. Roughly 2 weeks in between

- Hutton asked about getting rid of splitters

  - Probably can’t – need to match all the other optics still

  - If space and money are not concerns, anything is possible!

# Agenda topics

## Time allotted | 25 mins | Agenda topic Splitter Crosstalk| Presenter Donish

- Scott – not convinced crosstalk was cause of problems.

  - Need sufficient crosstalk management. With standard magnets, you have an expected pattern. Just manage it.

- Kirsten – crosstalk IOC tried to compensate. Wasn’t constant, but could have been worked out. Contributing problem, but bigger thing was that we “were flying blind”

  - Stephen – put hall probes in splitters to see what we have

  - Need more diagnostics

- Dejan – Adam was only using two magnets. Not following the optics. No one knows how he did it.

- Scott – at the last ERL workshop – Adam showed that by-and-large, he was really close to design optics by the end

  - Kirsten – we all got stuck on R2. All quads close except in R2 – that’s complicated.

- Dejan – interesting new development. New company called XLight. Bruce Dunam mentioned that there might be money for running CBETA again.

  - Also Chris Mayes, Georg, etc…

- Alex B – let’s disentangle from CBETA – murky

  - Scott – yes, murky

- Asked Donish to see what we can tolerate

  - Rematched the whole machine based on new LINACs

  - Right after spreader, there’s a 6 m drift before the next quad. Betas blow up huge

  - Using spreader quad – but didn’t help.

  - Will have to look into Arc1 lattice file to control betas

  - Might add a quad to help control it – there’s a 6 m drift

  - Preliminary study – field strength error on all quads and dipoles

  - Added energy and twiss errors as well

  - Using Dejan’s lattice

- Didn’t do all 6 lines. Focuesed on lowest E

  - Since beamlines aren’t finished, no correction applied

  - White is nominal, orange are “watch point”

  - Field strength error +/- 1%

  - 1% plays a slight role at exit of splitter line

  - Scanned energy error in steps

  - Much larger effect on output optics

  - Vertical beta explodes, huge dispersion leakage

- Reza – does 5% energy error allow the beam to get through the arcs?

  - Not sure

  - Dejan/Stephen – ought to go through the arcs

  - Reza – I mean through the whole machine

    - EM part of machine should be fine

    - Just looking at Splitters – good question. Didn’t add any correction, didn’t look at orbit. It’s too early at this stage without correctors

  - Reza – what is a reasonable energy error?

    - Got 5%  by dropping a cryomodule, multiplied by 4 just to give energy change of 4 passes as enter this beamline.

    - Reza – at this level now, we could not transport this without changing settings.

      - Halls talk about 10^-4 level energy. 5% is too much

- Salim – for the magnets – is 1e-3 the limit for magnet errors? Trying to understand.

  - Seems like a lot

  - Donish – trying to overestimate with worst case

- Ryan – when we have a cavity drop, we can operate by scaling magnets (LEM) and arc magnets.

  - Alex B – we can compensate

- Reza – would choose 2-3e^-3 to estimate how beam would go off. More than that, the rest of the machine would have too many problems.

- Alex B – we’ll have to evaluate adding correction to the splitters

  - Ryan – we have to build that in from the beginning.

- Remember, splitter lines aren’t done – this is just preliminary.

  - Reza – this is just a one-time error?

  - Are these studies concurrent, or separate?

- For each simulation, each magnet samples from a gaussian that is +/-1% at 10K runs. Unique set of errors on magnets.

- For energy, just a grid scan

  - Tested 10% errors on Twiss. It was an overestimate.

  - Grid scan on Twiss as well

    - Would be better to sample errors from random distribution, but for this preliminary study, focused on simple grid scan

    - Only 1000 this time

  - Alignment from gaussian – 10K runs

  - Doesn’t impact things too much. Alignment is hardly changing anything. Plays a bit of a role in Twiss input, but didn’t completely destroy the beam

    - Might be promising to give some flexibility in Transition

- Dejan – if we run CBETA, Stephen wants to put hall probes in each magnet and watch what happens in each magnet

  - Stephen – help with hysteresis also. And get an idea of crosstalk

    - Problems with repeatability

  - Dejan – had to ramp the magnets 5-6 times to get the right values

  - Stephen – reduced errors by 5-fold, but still not repeatable

  - Maybe hall probes would help.

- Ryan – did you have problems with power supplies?

  - Yes – looks like wrong ones bought, had to add capacitors to reduce oscillations, etc…

  - Circuit very unstable.

- Reza – in our machine, there are places with crosstalk. Two things we do:

  - Doing the order correctly (cycle them the right order and repeat)

  - For spectrometers, we go and put power supplies with that magnet and only do them with that PS and magnet.

- Dejan – in RHIC, when running low E, remnant field and hysteresis very important

- Ryan shares pictures:

    - Ryan’s hand is about 3.5” from outside to outside.

    - Septum on top, recirculated beam on bottom in dark blue

- Reza – one of biggest problems for crosstalk is not magnet to magnet, but the cable to the beam crosstalk.

  - The cables themselves impact the beam

  - Dejan – this happens a lot – important to remember that first ring was made of permanent magnets – couldn’t get beam through magnets b/c of impact of ring above.

    - Shielded with mu metal

    - Ryan: conversation with Jay, and apparently beam pipes made with layers of carbon steel with air gaps in between work great. Mu metal if more is needed.

- Alex B – thanks for starting this work, Donish

  - Thanks to all for bringing in more input

- Alex B – will have to look at which splitter – like the symmetric option

  - Ryan – yes, but that option doesn’t have two lines completed. Two lines are still shared, and do not have independent control. So that’ll change the layout once that is added.

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic AOB | Presenter All

- JLUO meeting next week.

- Alex B – maybe we only meet every two weeks instead of weekly for the summer.

  - Will resume in two weeks.

  - Dejan and Ryan agree

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g

Next meeting in two weeks. This will persist for summer (every other week).