# FFA@CEBAF Working Group|Minutes

## Meeting date | time 08/09/2024 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Reza, Edith, Salim, Kirsten, Alex C, Dejan, Donish, Stephen, |
| --- | --- |

# Intro Discussion

- Tropical Storm chat – bad weather

- Can we do multipass diagnostics? Topic today

# Agenda topics

## Time allotted | 25 mins | Agenda topic Notched Beam Diagnostics| Presenter Reza

- Quick understanding of general idea

  - NL and SL does this

  - Will need for FFA Arcs

  - Done at 60 and 30 Hz – assume here we’re talking about 60 Hz

  - Why? – first beam through machine, wanted to see the higher passes at the same pickup cavity

    - Two ways: send first then second and add together and find the difference between them

  - Alex B – revolution period is ~ 4 microseconds

  - If you make the signal slightly shorter, can tell different passes

  - Beam adds for 250 us, but at the 4.2 us, separates them out

  - Ryan – is this reading wiresum or current?

    - Reza – in the 250, average position of all, in 4.2, each pass – reading position

  - Reza –

    - 4 wires, difference over sum of currents

      - In 250, seeing sum, in 4.2, seeing difference over sum

  - This is NOT CW in this case.

  - Gun at 499 MHz, - that’s CW

  - All need is that 4.2 pulse added or subtracted from CW

  - Results would be a snake-beam or ripple on system going around at 4.2 or 5.37 us, and would show up as X pulse

  - If subtract from CW, get negative position

  - Can add or subtract, would get these signals

  - Add or subtract from average

  - Alex B – what frequency would we do that at? 60 Hz?

    - Reza – TBD. Would have to decide the best way to handle this and not interfere with other diagnostics

    - Adjustable – must decide

- Reza – where do you put it in the 60 Hz?

  - Beam sync sets everything else in the machine

  - BPM knows have to go 250+100+4.2+rest of time each cycle

  - Gun group – problem with notch at beam sync, but can put anywhere in 60 Hz pulse

  - If notch too small + or -, BPM may not handle it

    - 5 uA is amount of beam usually for this

    - So if we do current run (let’s say 80 uA for example), could add/subtract 5 uA

    - Timing can be adjusted for laser and gun requirements

    - Prefer subtracting – total current limited due to RF limitations (we often run at the very edge of operations ability)

    - Can also use for measuring/monitoring pathlength in CW

      - Cavity in beam line – measure timing of beam. If the notch/pulse goes through, path length can be determined in CW

    - Should work in NL/SL and FFA arcs

- Ryan – question – our BPMs are super noisy – hard to determine perfect diff/sum or position

  - Could add in wiresum for additional data point to help clean signal

    - Think see notch, but wiresum could help see if it’s noise, or real signal

  - Reza – self-test with wiresum in current BPMs for lock BPMs for example

    - Depends on sensitivity of BPMs

    - New BPMs should be more sensitive

    - Right now, 5 uA of beam in pulse mode, can see it

  - Ryan – adding/subtracting 5 uA on the CW could be lost in noise from the main signal. Doable, but need to consider noise in CW.

    - Sensitivity issue. 60ish uA, then +/- 5uA on that could be hard to read

    - Kirsten – worried about laser modulation? Or noise?

    - Reza – good point

    - Ryan – this is very doable, but have to think of how to clean signals to make sure we’re reading the right thing.

    - Alex B – instrumentation group can probably help with that.

- Alex B – good to get path length – can also get info on momentum compaction

  - Can use a cavity to kick the beam in energy, extract from the timing before/after kick to get momentum compaction

- Reza – yes, all the things we can do in tune mode we should hopefully be able to do live in CW

  - Go to tune mode for machine safety

- Alex B – you spoke to users?

  - Reza – spoke with physics users, the people there said for detector it doesn’t matter, just summing up events.

    - Want to check with more users to make sure

    - If it’s off for a while, then they’re in trouble – trying to make this as short (4.2 us) and limit amplitude just in case

      - Not sure if it’s important, but 10% up/down should be fine probably

- Dejan – when designing eRHIC ERL, we had always assumed there will be a special electron calibration pulse in front of the other beams to use it to remove all the misalignments, etc…

  - In CBETA, wanted to be sure we could see different pulses with same BPMs  - was a problem for a while

    - By programming laser at source, can always have special pulse

    - Saw them independently very well in the end – but had problems in the beginning

    - BPMs didn’t need special pulse

  - Kirsten – pilot bunch (never got around), but operationally

    - Basically limited number of bunch pulses per each train so you could have all the trains stacked up without overlapping

      - Made really short trains with almost no gap (tiny)

      - Window the BPM time slot for readings

    - Reza – so you weren’t running “real” CW?

      - Right

  - Pilot bunch was similar idea that you mentioned, but never implemented.

- Reza – in pulse mode, you can either look at steps, or at 4.2 us spot

  - Looking at steps needs some math, and has interactions

  - With just CW, just a line, can’t get information without disturbing CW

- Ryan – you spoke with Nate?

  - Yep, and new BPMs will be better

- Alex B – please put the slides in the folder or send to Alex

- Dejan –

  - Problem we were having:

    - Adjust things “in the box”

    - Kirsten – each color is a different signal

    - Instead of taking the whole area, you only take the area of the calculation to one of those passes

  - Dejan – couldn’t get last pass – signal was too weak (7th pass)

    - Kirsten – b/c we lost a lot of beam

- Reza – going to ask gun group if they can make this beam, or something similar

  - Then do beam test

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 25 mins | Agenda topic AOB | Presenter All

- Deadline on 27th of August for LOI for SBIR

  - Can go with Saber or other PA company

- Stephen – do we have to pick?

- Dejan – we were discussing

- Competitive bids? Choose company?

- Prepare proposal or LOI with them

- One company merged

- Get JLab to submit proposal with Saber to build magnets

  - Decide on types of magnets

  - When passes by, can get them built and measured – big deal

- Alex B – you have a good track record at BNL – when do you think better to submit? Submit from BNL instead of JLab?

  - Spoke with M. Shinn – need to get the right solicitation

- Alex B – we’ll get the magnets, company will get money

- Dejan – Stephen and Dejan write LOI, send to JLab b/c it’s the CEBAF upgrade

  - See if we all agree on the LOI, we sign it, send it to DOE

  - Alex B – BNL has the expertise, and can guide company

- Alex B – 27th is the LOI deadline? I think we should go for it

  - Will get in touch with Andrei

- Stephen – what has to know what is in it, who to sign, etc…

- Alex – maybe Mike Spata?

- Ryan – have you spoken to Kevin Jordan, who has had tons of these?

- Dejan – last company was horrible. Let them design. If we give design, maybe it’ll be better

- Discussion about companies and options

  - Eligible vs small vs ability, etc…

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g

Next meeting in two weeks. This will persist for summer (every other week).