# FFA@CEBAF Working Group|Minutes

## Meeting date | time 10/11/2024 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Randy, Vasiliy, Nick, Scott, Salim, Dejan, Reza, Donish, Stephen, Edy, Andrei, Tim |
| --- | --- |

# Intro Discussion

- Weather talk

- Nick Sereno joins

# Agenda topics

## Time allotted | 50 mins | Agenda topic Transition| Presenter Randy/Vasiliy

  - This is from just before IPAC

  - Was gone for about a month – not much change since then

  - After the last ARC dipole, have about 15 m before the recombiner

  - If you only use the adiabatic match, it doesn’t work – moves outward toward wall (top right plot)

  - 16 cells to bring them together, 10 cells to adiabatically match

    - Rough – doesn’t quite match or hit, but good start

  - Not enough space to only use adiabatic matching – can’t get orbit and dispersion to close dispersion

  - Adiabatic first, then harmonic to get them to match

  - Sort of got Beta_x, but dispersion is being pushed back up and the orbits are hitting apertures

  - Adding in 2nd pass makes this worse

  - One issue: if we had correctors in non-dispersive area it would work better

    - If no bending, it might work – but no choice

    - Only last cell has no bending

  - Orbits are crossing over the apertures

  - Second method might not work b/c geometry

    - Basically move things upstream to get a 15 m straight section – but this only gives about 5 cells

  - Maybe only send orbits and dispersion set, and allow Twiss to just go through LINACs uncorrected

    - See if splitters can handle the re-matching then

- Stephen – what was the correctors used?

  - Just a thin corrector in between FODO cells

  - So weak quad?

    - Yes – dipole for orbit and disp, quad for betas

  - Beta – normally you have to change the cell lengths

    - So make magnets longer with weaker magnets – will give higher betas

    - If using only the strong magnets we have now, you’ll probably get betas oscillating rapidly

  - Randy – yes, that would work. But the matching into very specific betas is hard.

    - Matching to 6 specific betas/alphas is very hard, but your method would help

    - It might be helpful to let splitters do the matching, and allow the Twiss transport “as is” in LINACs

  - Stephen – tiny area and sensitive match

- Dejan shares:

    - Problem with betas

    - Red arcs show space required for this

    - This uses power law – lots of space taken here

    - Our betas are ~120 m – need sells probably almost twice of this size

    - Maybe need something other than power law so it can go faster on merging orbits

- Alex B – 5 cell transition doesn’t give enough DoF

  - So if you need to go back into the arc, that’s doable.

  - Randy – no, 5 cells isn’t the transition itself.

    - So start adiabatic part earlier and have the ~15 m length to bring down the Twiss

    - However, there’s no space left

- Dejan – cut from 90 degrees to 83 to allow for adiabatic

  - Randy – yes, but if you’re not straight at the end, can’t align with recombiner – MUST BE STRAIGHT

  - Yes but maybe go to 75 degrees instead of 83

  - Stephen – that’s what he’s done already

  - Narrow line is only the free space available

- Stephen – longer is tricky b/c dispersion and orbits wander

  - Can put less focusing in the cells – just add in less magnet

  - Reduce F and D in them

- Dejan – can’t merge without reducing bending

  - Go back to first picture:

  - Instead of using same cell length, make the cells longer and repeat like below

  - Randy – will try

- Stephen – but we don’t have much space to lengthen cells. But maybe smarter to reduce fields in them

- Vasiliy – cell length changing only works if phase advance per cell is roughly fixed

  - 20 m beta cell has very little phase advance

  - When going from short to long cell, can’t keep phase advance

  - Stephen – get too much focusing – need to reduce field too

  - Vasiliy – tried this but didn’t work

    - Used large number of cells – didn’t work. Shouldn’t abandon yet, but will likely require a large amount of space to go from several meters up to over 100 m

- Dejan – changed bend, length, and gradients

- Vasiliy – gradients have to be strong enough to keep phase advance constant

  - Stephen – don’t want PA to go up and become unstable, but maybe can send it down

    - Might get bigger betas for optimizer

  - But that’s not systematic – think tried in past and wasn’t successful

    - Dropping PA requires brute force optimizing

- Dejan – main law used was power law – but maybe there’s a third order or some other way to reduce length of matching and still be adiabatic

- Vasiliy – only two options for systematic match:

  - Adiabatic with cell length

  - Harmonic match – might need to go through several focal points

- Vasiliy shares:

  - Limited space, adiabatic suppression of orbits and dispersion need to be done in a small number of cells (10ish)

  - You get close, but low E orbit not suppressed perfectly (lower left plot)

  - Harmonics used for quad and dipole components at the betatron tune frequencies

    - Each family controls only its own energy

    - Magnets modulated at frequency of betatron tune

  - Lowest E dispersion is concerning (lower right) – way to improve?

  - Tried resonant harmonics for passes 1 and 6

  - Able to generate 50 m betas at the end of the matching for passes 1 and 6

  - Kept control of the rest of the passes mostly

  - Pass 6 look OK, but pass 1 really starts oscillating heavily (top left plot)

- Any way to move beta wave later (downstream)?

- Is resonance throughout all of matching section for quads right?

  - When they’re introduced, they make a dipole component on all passes from feed-down of quad field

- Introduced dipole component at the same frequency and tune as the modulation of the parametric resonance quad harmonic

  - This may have been why controlled a little better

  - Tried using only the last 10 resonance quads and look at lowest E pass

  - Get modulated beta wave (top left plot)

  - Not sure why modulation is happening

  - Looking at highest E pass, used all 28 quads and dipoles of same harmonic

  - Able to excite betas for highest E up to 50 m, but it also impacts lower E passes

  - Maybe power every few cells for certain energies?

  - Corrector families have non-trivial interactions with resonant harmonics

    - Right now 1 family per energy, maybe change that

- Stephen – have you assumed any limits on quad correctors?

  - No, not yet. Looking for any solution first

  - Stephen – can go downwards a lot if you take magnet away, though you also remove dipole then

- Easy way to change quad a lot in the last few cells, reduce length (but also then lose dipole)

- Can also play with lengths and positions

- So can play with strengths of main magnets – change drifts between, and change lengths pretty freely

  - Vasiliy: For the model, keep the length the same and alter the strength

  - Stephen – might not want to do that b/c interplay

- Vasiliy – if we squeeze things upstream a bit, maybe gain some real estate

  - All for increasing cell length, but maybe not space

- Stephen – your corrector is in a fixed position in the cell?

  - Yes

  - Introduce more drifts and shift magnets back and forth on the cell?

- Vasiliy – more considering positioning of the entire family of magnets

  - Stephen – but yes, what if you change where in the cell these corrections take place?

- Vasiliy – maybe let correctors vary independently based on betatron tune

  - Stephen – b/c it’s so short, you’ll need to optimize all the variables you can – adiabatic only rough approx. at this point.

- Ryan – how many magnets per cell?

  - Vasiliy – one Y and one X (one on the F or D mags)

- Alex B – steering/modulation?

  - Tried to compensate steering by introducing dipole at the same frequency as quad component

- Dejan shares:

  - FFA in end.

  - With only 2 magnets (combined function) can merge – can be done analytically

- Vasiliy – we tried the short-quick

  - Worked for orbits and dispersion, but not beta

  - Maybe combine?

  - Use short section at end for orbit and dispersion and use several cells before final cell to get betas?

- Alex B – optimizers from linac optimized some way

  - If you find a more preferred configuration of the Twiss, can redo things for multipass optics for linacs

  - Probably need to give us a hint in which direction to go

- Dejan – if I raise the linac and start with the first module

  - Keep tunes same in every module, then raise gradients as accelerate, then need higher gradients for phase difference

    - Track multiple turns – have you done it?

  - Alex B – yes, we’ve done this. Only visible for first linac – not detrimental

    - Keep phase advance constant – but regulate end field focusing with quads

    - Optics – transparent – same PA

  - If LINAC is drift, get 150 m beta

  - Must be optimum value by using the LINAC as the IR

    - Alex B – can have multipass optics to have quasiperiodic solution for all passes

      - But betas gradually increases (thinking 167 m for top pass)

  - Dejan – I got 110 m – depends what beta you choose in the middle

  - Alex B – if you have the whole chain and not just a single cell, changes

    - At the 22 GeV, getting elevated to ~160 m

    - Dejan – we should revisit

    - We did this, then went back to strong focusing to get Betas and Alphas right

- Dejan – because the quads inside of the LINAC don’t do much at higher Es, can play with triplets at end, focusing with low and high E optics

- Alex B – Donish made the code a bit modular so we can drop in different versions of things.

- Please put slides in the folder!

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g