# FFA@CEBAF Working Group|Minutes

## Meeting date | time 03/14/2025 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Ryan, Volker, Edy, Scott, Dejan, Stephen, Kirsten, Salim, Donish, Todd, Roger, Randika, |
| --- | --- |

# Intro Discussion

- CEBAF operational woes – broken LCW pipes, etc…

- Salim will aim for an LDRD on Kirsten, Ryan, Jay, and Kitty’s idea from a few years ago at the retreat.

  - There are tech notes, slides with costing, etc…

# Agenda topics

## Time allotted | 50 mins | Agenda topic Field Map Measurements| Presenter Volker

  - Essentially, wiggling the beam

  - Record positions, add noise

  - Algorithm gets the good stuff from each

  - G is a transfer matrix – calculate auxiliary parameter

  - Eye is a unit matrix in MATLAB

  - Qhat – update the best transfer matrix element guess after the new iteration arrives

  - Copy new P to old P

  - P is 4 x 4 matrix

  - Do identification error (last equation) – Rt is best guess

  - Factor 100 in variation vs RMS noise on BPMs

  - Drops as 1/T

  - ID error is red – on order of unity

  - True transfer matrix is R (not a quad, just a random matrix with determinant = unity)

  - Simulation takes no time at all

  - After 10K iterations (orbits)

  - Can do this analysis anywhere we have input and output

- Stephen – can you generalize if you have nonlinear transport?

  - Around slide 5 – when you do this in a clever way, you might be able to accommodate nonlinear mapping?

  - Volker – T matrices?

  - Yes

  - If you express it the right way around, comes out as a linear system still.

    - G would contain all the monomials

  - Volker – hadn’t thought about nonlinear yet – interesting

- Stephen – have considered nonlinear magnets, but also would be nice to quantify “accidental nonlinearities”

- Scott – you’re basically treating this as a least squares system, can generalize to higher-order

  - Feel like this is doing it the hard way. In principle, when you have a least squares system, just solve it numerically with Qr

  - This will give the solution, the RMS errors, and the error propagation is straightforward

  - Have to be a little careful with errors, but you’ll get rid of inverses, etc…

  - This is sort of a one-shot with all the data

    - Volker – yes, if you accumulate all data, then solve with eq on slide 6

    - If you have continuous data, fold it into best fit system

  - Basically, you just work with a rectangular matrix. Factorize.

  - Volker – which rect. Matrix? Scott – U is rectangular

    - No, U is a column vector (each G is 2 x 4)

    - Large number of rows, small number of columns, QR algorithm solves it without the UtU

    - Result lets you compute everything you need

- Dejan – We have a system, and have to do exactly what you’re looking at.

  - This is exactly what Scott is saying. Approach the problem from going through repetitive measurements, or get all the measurements at once and do LS.

- Scott – this has some tricks where you could “get it as you go”

  - One reason QR is used is b/c forming the companion matrix is “numerically fussy”

  - Volker – start with a unit matrix guess, then change as you get more data

  - Scott – what I’m saying is that you’re still forming the companion matrix, even if it’s through iterations. Our problem isn’t really numerically fussy, so probably not a problem. But however you make the companion matrices, it can make problems if there is fussy numerical problems.

- Stephen – I like SVD on rectangular matrices for stuff like this.

  - SVD is QR on steroids – doesn’t make much sense here – too much. If you start dropping off some of the coefficients, you’d have issues.

  - Stephen – so delete all the small coefs, and the biggest ones give you the LSF

    - Scott – can’t do that here, need all the coefs

- Dejan – we do have a nonlinear system with different energies into the input

  - We have very nonlinear magnets for each energy’s mag field

  - This way, we can find out the errors in the multipoles coming out

  - Volker – the nonlinearities in the phase space coords? – So higher-order depending on other monomials

- Volker – will make github with MATLAB so we can play

- Alex will add things into presentation folder

- Salim – was going to ask about the error – but that’s answered

  - Now thinking, what about adding misalignments, etc… Will this determine R correctly?

  - Volker – you wiggle the orbit around some initial reference, and then make a tangent map around orbit you had when you started

    - Basically a linearized map around where it’s varying.

  - Salim – calculating an R matrix with dipoles and quads – but in return, I’ll get one exact value for each element. If the beam is off-axis, and the BPM which is measuring is also off-axis, at what percentage is this reliable?

  - Volker – you address an ambiguous point – in the end a dipole is a dipole, but one job is to change reference trajectory, but also the motion around the reference trajectory.

    - Global offsets related to job of the dipole as a defining element of reference energy

  - Salim – the “black box” we’re looking at, it’s one box, and we may have many other terms inside

  - Volker – this addresses the transfer matrix only

- Ryan – this is similar to how we do some things in CEBAF – we start with absolute orbits, but then once you’re set up, you re-zero everything and have relative orbits

- Alex – I recall Alex Coxe having issues displaying things from patches. I think it was something about how Bmad was handling the transition from the strong-to-weak focusing?

  - Ryan – no, it’s more about redefining the local coordinate system. So you’ll have a place that has a reading that is then reading zero, b/c the coordinate system shifted.

- If you put in the “forgetting factor” with 20-30 minutes and get good variation, you can pull out the time scales. Can track as you go along. Adapts to slow parameters

- Ryan – can use that for a slow feedback system

  - Volker – this uses the idea that the orbit feedback system – reconstructs the response matrix. Use LOCO-type analysis

  - Accumulate FB data and parasitically extract cavity parameters

- Recursive least squares is “Volker’s Hammer”

- Alex – also Volker has an accelerator physics text using MATLAB

  - Open access (oapen.org) and publisher’s website

- Dejan has Bmad again – can do translations.

- Dejan – what about support from JLab?

  - Todd – have to look at how the CR vote goes.

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

- N/A

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g

No meeting next week!