# FFA@CEBAF Working Group|Minutes

## Meeting date | time 04/04/2025 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Ryan. ### Timekeeper; Alex B. | Attendees Alex B, Donish, Ryan, Edith, Volker, Scott, Kirsten, Nick, Salim, Andrei, Dejan, |
| --- | --- |

# Intro Discussion

- LDRD announcement just made

# Agenda topics

## Time allotted | 50 mins | Agenda topic Sym. Splitter | Presenter Donish

  - Very tedious process. Iterative work

  - Problem is b/c of the small betas at FFA interface, system is very sensitive to parameter changes (quad strengths for example)

  - Latter steps oscillate (right side of plot). Optimization takes a long time. This is what kills the progress.

  - To improve, reduced the step sizes – slows down simulation run time

    - No guarantee of success

  - Match backwards

  - Create generic and symmetric bend system. Beam goes from FFA to LINAC end.

  - Top plot is not a typo – make a periodic system with only FFA betas.

    - This way make an arrangement of magnets that looks reasonable with enough versatility to match FFA to FFA

    - Should get decent layout, and then perturb linac side

  - Using periodic solution, drop the FFA matching conditions on the LINAC side (right side here). Match from BetaFFA to BetaLINAC

    - Reverse system and get the Linac > Splitter > FFA optics

  - Need enough drift space to fit magnets WRT other beamlines

  - Beams exit the first splitter at different angles and positions

  - Ryan – cool to do this based on dispersion instead of magnet sizes and strengths, etc

    - Donish – lengths and strengths will come out if you define the etas.

  - This is symmetric, BetaFFA matched to BetaFFA

  - Soft constraint – fully aware this is not a solution we want or can be used.

  - Don’t want to optimize this. Just a generic magnet layout

  - Alex – this is mirror symmetric?

    - Yes

  - Triplets, doublet in between first and second dipoles

  - Dejan – this is too many magnets to do the simple problem.

    - 6 center dipoles added for R56 flexibility. 4 didn’t do the job

  - Dejan – provided an example of the simplest possible solution for the chicanes

  - Changing the periodic solution, perturb magnets to match into BetaLINAC

  - Not soft constraints.

  - Start sequentially backwards. Increase number of quads from LINAC side

  - Start with at least 4 quads, see if match into LINAC Twiss

  - Work backwards, keep optics created in periodic cell, and only perturb on linac side until match

  - 0.07 m R56 is the “old” values (from standard FFA Arc). 0.0 m R56 is for possible isochronous arc

  - Matching into the Twiss aren’t an issue

  - Difficulty is getting everything matched

  - Ryan – you can probably lower the weight on R56 since we don’t really know what it’ll be in the end.

  - If try to get R56, betas get very large

  - Which quads can be used for R56 knob (orthogonal to Twiss knobs)?

  - Not clear analytically where to put it

  - Took beamline, in drifts injected small thin quads, then calculated the effect of it’s impact on betas in both planes and R56 as you change strength of quad

  - This is a map where it shows the effect of any given inserted quad

  - Was hoping for a place where they’d be completely orthogonal

  - Kind of see it at 28ish meters – all lines go to zero except the purple/blue trace – therefore vertical beta would be impacted the most.

  - Green R56 line never has a separate place – it’s always coupled

  - System is so coupled, so any changes destroy what you’ve done

- Scott – you’re heading the right direction.

  - Don’t add quads. Take the quads you have. Forget R56, look at nice solution. Now look to change R56 in whatever direction you want to go.

  - Take constraints and derivatives of constraints WRT variables – get rectangular matrix

  - QR algorithm applied to matrix. You’ll find two groups of orthogonal vectors

  - You’ll have a rectangular R matrix (QR algorithm transposed – PLQ decomposition)

  - PLQ – L matrix is also rectangular. Top block has nonzero values. That’s the bit that gives you the matrix that you effectively invert to have a change in constraints –

    - Invert nonzero block at top of L matrix

  - Right half of L matrix are zeros. That’s the part where you’re not doing anything

    - They don’t impact the Twiss. It’s a space of vectors that won’t affect matching, but WILL impact other things, such as R56. Now you can move around that space.

  - If you’re feeling clever – extend that to find the vector that maximally changes R56 without touching the rest.

  - Here’s the catch: this is very very local. Things are so wacky here.

    - How change R56 by a mm? Things will move

    - Do a small tweak to re-fit the betas again, take the derivative again, then take the next step

  - This is how did things in CBETA – but there had the advantage of only having two extra quads.

  - Would do it in 2 steps: take current R56 as constraint – scan around that space to get Betas (Twiss) as close as possible

    - Step up R56 again, rescan, repeat

  - More quads, more complicated

  - You have to take small steps, refit, find new space, repeat

- Donish – I’ve been seeing that too. Iterative process. Like Scott’s idea

- Dejan – you have to understand what is R56 – it’s momentum compaction. Integral of dispersion over bending radius

  - When you do anything with matching. Have to draw vectors of dispersion to see where it’s at

  - All parameters and matrices are following that.

  - Anything you do, draw the picture of the dispersion function vectors.

  - Dejan – all these matches are fine, but need to see picture of what you’re doing.

  - Donish – yes, I looked into that. Never really used the normalized momentum space before to design them.

  - Lower left plot (yogi)

  - Looked into it, read the papers – it’s a bit ambiguous to Donish to make the connections

  - Dejan – vertical is dispersion/sqrtBeta…describes work

  - Scott – plot a whole lot of intermediate points, especially in the drifts

    - In the drifts, anywhere outside of dipoles, going on arc of circle in plane

      - Having a ton of points in the plot, you’ll see those arc circles, and see where the radius is changing

    - Whenever radius changes, it’s a dipole

  - Very helpful in looking at these things.

- Scott – other hint: when you go through this stepwise process, and trying to step up the R56 a mm at a time, at some point, it’s just going to fail.

  - Why? – look at the plot. You’ll see better with more steps.

  - Basically, making a certain number of turns around this.

  - You’ve heard discussions of x/Pi solutions – this is basically that

  - In the parts without dipoles, advances with phase advance

  - You’ll find that you have solutions with phase advances around Pi. Can’t get solutions from ~Pi to ~3Pi

  - Beyond range, you’ll need a completely new solution with more turns

    - Need more or less phase advance

- Donish – If you look at 0, 0 – coming from nonzero dispersion

    - Scott drew on the screen to show what more points would do

- Ryan missed a few minutes (school nurse call)

- Donish question:

    - What would you calculate here?

    - Let’s say 45 degrees:

    - Quads define phase difference, not dipoles

- Donish – the plots he read are for periodic systems. They look nicer.

  - Our system is not periodic.

- Scott – when talk about phase advance, ignore last jumps from dipoles. Don’t worry about the first point near zero. Start at the first point on the curve when counting phase advance.

    - From elbow to elbow

    - Is that 3Pi?

      - Maybe

- Can phase advance go backwards?

  - Maybe, not usually

  - Look at the “knee” on the yogi – that’s a dipiole. Phase advance going “backwards” but not really

  - PA is only approximatation of PA in this diagram

- Take what you have, find out the R56, and don’t force it. Walk it.

- Ryan- extraction? Offset, x’, ToF?

  - Donish – not yet. It boggles my mind on how to include that.

  - Dejan – start with ToF, create values of time differences

  - Donish – Ryan means adjustable ToF

  - Kirsten – sometimes you can use magnets, sometimes you need movers

    - Even with some movers, it can be a headache

  - Ryan – we do have doglegs

  - Scott – don’t worry about ToF – the big thing was getting the matching.

    - Ryan – I think we have to “bake it in” from the beginning b/c we move a lot more

    - Kirsten – I think it’s less true for CEBAF than CBETA

    - Scott – RF wavelength similar

      - Do I have to hit a target now? No. Hit it later

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Time allotted | 10 mins | Agenda topic AOB | Presenter All

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g