# FFA@CEBAF Working Group|Minutes

## Meeting date | time 11/21/2025 | 11 AM EST | Meeting location https://jlab-org.zoomgov.com/j/1614898082?pwd=TnUzMS81M2sxbDZIbERJU01tYkJCQT09

| ### Meeting called by; Alex B. ### Type of meeting; Weekly Meeting. ### Facilitator; Alex B. ### Note taker; Salim. | Attendees Alex B, Donish, Salim, Dejan, Stephen, Leon, Kirsten, Volker, Kirsten, Sadiq, Vasiliy, Scott, Andrei |
| --- | --- |

# INTRODUCTION

- Leon Van RISSEN-HAUPT is being introduced to the group.

- Alex said CBETA magnets can be taken. Stephen said he has a full magnet, and he recommends caution with such magnets.

# PResentations

- Revisiting hybrid-FFA Splitter design - Donish Khan

# Agenda topics

## Time allotted | 30 mins | Agenda topic Revisiting hybrid-FFA Splitter design| Presenter Donish

- Donish starts with simple chicane and adds FFA magnets where he divides the chicane into 3 sections.

  - Stephen wrote to the chat: “You can get an idea of the size of permanent magnets using this https://stephenbrooks.org/ap/halbacharea/ “

Donish: Middle figure has typo in the title: it should be vertical beta function and rightmost plot should have the title horizontal dispersion.

- Alex B: which optimizer are you using? Donish: Elegant’s simplex method.

- Dejan: you can use the normalized dispersion and check if 360 deg rotation is achieved.

- Stephen: you haven’t assumed the beams are in the same pipe yet? Donish affirmative.

- Dejan: the gradient limit for the magnets? Donish: 53 T/m for the quads, 1 T limit for the dipole.

- Dejan recommends choosing one energy, the most challenging one, and focusing on its optimization by varying the combined function magnet gradient changes.

- Scott: use also the FFA magnets to help you in achieving the smaller beta functions. Donish: not yet done.

- Andrei: the optimizer is optimizing the beta functions and orbit/trajectory? Donish: both. Andrei: are you clustering the distinct energies? Pushing certain energies to certain trajectories? Donish only the lowest energy blue line is pushed.

- Stefan: as done in the muon collider, maybe the triplet can be used better control of dispersion and beta functions.

- Leon: constraint the optimization for example 5-pi phase advances that would automatically close the dispersion and relax the beta function. Donish: it was recommended in the past and can be implemented now.

- Salim: the FFA magnets in the optimizer are allowed to switch the quad gradients to be defocusing and focusing without any quads being imposed to be (de)focusing? Donish affirmative: the optimizer decides and discovers the sign. Salim says partially the triplet structure is automatically explored, then but not fully.

## Time allotted | 30 mins | Agenda topic Title Update| Presenter Person2

| Action Items | Person responsible | Deadline |
| --- | --- | --- |

## Special notes

Pathway to Repository: https://jeffersonlab-my.sharepoint.com/:f:/g/personal/tristan_jlab_org/EqZ5MeS-nipCgPfZB5p0oS4B9Is67d3nQb9sLJI3Zyev9g