BELOW-THE-HOOK LIFTING DEVICES

INSPECTOR  Doug Tilles          INSPECTION DATE __3/31/2010

NOTE: 	1.  Proof test to 125% of rated capacity.  The test load shall be accurate to 		     within -5%, + 0% of stipulated values.

2.  Qualified Inspector shall witness all steps below.

_Rig and lift 2500 lbs @ of WL 125% _______________________________________________________________________________________________________________________________________________

Examination

Lifting bars and spreaders shall be checked for signs of incipient failure in bending and shall be replaced if permanently bent more than 1/2 in. in 10ft. or twisted more than 5 degrees out of the original plane.  Hook attachment welds shall be examined for cracks and signs of failure in tension.

Qualified Examiner shall perform test by visual examination, liquid examination, or mag. particle examination.

Acceptance:  No cracks, linear indication, laps, or seams.

STATIC TEST:  Hold weight for 10 minutes and visually inspect for deformation.

Type _shield block__  Size 2400 lbs +__4    25 lbs_lead bricks

Rated Capacity (SWL) _2000   lbs.  Actual Load Test 2500  lbs.

Serial Number ______________________

Qualified Inspector Verify (Load Test)       Doug Tilles

Remarks  Hoist Make Coffing

Model LHH

Serial # QY0033

There was no changes in the beam at any locations. The load was moved from one end to the other and stopped 2 ft increments.