|  |  | TITLE: | ES&H Manual |
| --- | --- | --- | --- |
| DOCUMENT ID: | 3310 Appendix T2 Operational Safety Procedure Form |  |  |

|  | Serial Number: |  |  |  |  |  |  |  |  |  |  | [Enter Serial Number] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | (Assigned by ESH&Q Document Control x7277) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | X | OSP |  |  |  |  |  |  | TOSP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | *Attach the Task Hazard Analysis (THA) related to this procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Issue Date: |  |  |  |  | June 15, 2011 |  |  |  |  |  |  |  |  |  | Expiration Date: |  |  |  |  |  | June 14, 2014 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (No more than three years from Issue Date except TOSP which is three months from issue date) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Title: |  | “Skill of Craft” work under conditions that require use of fall protection equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Location: |  |  |  | Physics division areas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Risk classification (See ESH&Q Manual Chapter 3210 Appendix T3 Risk Code Assignment.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Without mitigation measures (3 or 4): |  |  |  |  |  |  |  |  |  | 4 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | With mitigation measures in place (0, 1, or 2): |  |  |  |  |  |  |  |  |  | 1 |  |  |  |
|  | Document Owner(s): |  |  |  |  |  |  |  | [Enter Author's Name] |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | June 15, 2011 |  |  |  |  |  |  |  |  |  |
| Supplemental Technical Validations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hazard Reviewed (per ES&H Manual 2410-T1): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Subject Matter Experts Signature: |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |
|  | Dave Kausch – Aerial Platform |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ned Walker – Fall Protection & Ladder Safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bert Manzlak – Division ESH&Q Liaison |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approval Signatures: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Print Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |
|  | Division Safety Officer: |  |  |  |  |  |  |  |  | Javier Gomez (Acting) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Department or Group Head: |  |  |  |  |  |  |  |  |  |  |  |  | Rolf Ent (Acting) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Safety Warden of Area: |  |  |  |  |  |  |  |  |  | [Enter Safety Warden's Name] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other Approval(s): |  |  |  |  |  |  | Robert Michaels – Acting Hall A Leader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Volker Burkert – Hall B Leader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Stephen Wood – Hall C Leader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Eugene Chudakov – Hall D Leader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Christopher Keith – Target Group Leader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Document History: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Revision: |  |  | Reason for revision or update: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Serial number of superseded document |  |  |  |  |  |  |  |  |  |  |
|  | [Rev] |  |  | [Enter Reason] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [Previous Rev] |  |  |  |  |  |  |  |  |  |  |

Distribution: Copies to: affected area, authors, Division Safety Officer, ESH&Q Document Control

After expiration: Forward original and log sheet of trained personnel to ESH&Q Document Control.

|  | - Purpose of the Procedure |  |  |
| --- | --- | --- | --- |
|  | Be able to perform ``Skill of the Craft’’ work under conditions that require use of active fall protection. |  |  |
|  | - Scope – include operations, people, and/or areas where procedure applies |  |  |
|  | Members of the Physics division authorized in this OSP are allowed to perform ``Skill of the Craft’’ work under conditions that require use of active fall protection throughout the Physics division areas. This OSP cannot be used for planned work that retains a risk code higher than 2 after applying the mitigating measures described in this OSP. The applicability of this OSP cannot be extended or supplemented by any other document. A new OSP or TOSP must be developed for that work. |  |  |
|  | - Description of the Facility: (include floor plans and layout of a typical experiment or operation) |  |  |
|  | Experimental Halls, Physics Storage Building, and any Physics Division controlled spaces in other buildings, for example: EEL, Test Lab, etc., where fall protection is required. |  |  |
|  | - Authority and Responsibility: |  |  |
|  |  | 4.1 Who has authority to implement/terminate |  |
|  | Authority to implement: The supervisor of the work. In the Experimental Halls, this is the Hall Work Coordinator. Authority to terminate: Division Safety Officer, or anyone under the provisions of Chapter 3330 of the EH&S Manual. |  |  |
|  |  | 4.2 Who is responsible for key tasks |  |
|  | ESH&Q Chapter 6131 lists specific responsibilities of the personnel using the fall protection, their supervisors & of the Material Handling Safety Representative |  |  |
|  | - Who analyzes the special or unusual hazards (See ES&H Manual Chapter 3210 Appendix T1 Work Planning, Control, and Authorization Procedure) |  |  |
|  | Work supervisors, including Hall Work Coordinators, SOTRs, and group leaders listed in this OSP. |  |  |
|  | - Personal and environmental hazard controls including: |  |  |
|  |  | 6.1 Shielding |  |
|  | N/A |  |  |
|  |  | 6.2 Interlocks |  |
|  | N/A |  |  |
|  |  | - Other |  |
|  | Survey work area for potential electrical hazards (e.g. power lines) – minimum distance is 10 ft. If necessary, follow procedures outlined on Chapter 6110 “Lock, Tag, Try (LTT)” to bring energy sources/equipment to a safe state before proceeding with the elevated platform work. |  |  |
|  | - Monitoring systems |  |  |
|  | N/A |  |  |
|  | - Ventilation |  |  |
|  | N/A |  |  |
|  | - List of safety equipment (i.e: personal protective equipment or special tools) |  |  |
|  | EHS&Q Chapter 6131, Appendix T2, describes the types, components and care of active fall protection systems. |  |  |
|  | - Associated administrative procedures |  |  |
|  | The following EHS&Q chapters and their associated appendices apply to this OSP and are required reading: - 6131, “Fall Protection Systems” - 6132, “Ladder and Scaffold Safety” - 6147 “Aerial Work Platforms” |  |  |
|  | - Operating guidelines |  |  |
|  | Equipment manual |  |  |
|  | - Notification of Affected Personnel (How and Who) |  |  |
|  | Read & sign this OSP |  |  |
|  | - List of steps required to execute the procedure from start to finish. |  |  |
|  | The procedures to inspect and use the fall protection equipment are described on ESH&Q Manual Chapter 6131& appendixes. The worker must also identify the approved anchor points to be used before starting work. |  |  |
|  | - Back out procedures, i.e., steps necessary to restore the equipment/area to a safe level. |  |  |
|  | N/A |  |  |
|  | - Special environmental control requirements: |  |  |
|  | N/A |  |  |
|  | - Environmental Impacts (See EMP-04 Project/Activity/Experiment Environmental Review |  |  |
|  | N/A |  |  |
|  | - Abatement Steps – Secondary Containment, or Special Packaging requirements |  |  |
|  | N/A |  |  |
|  | - Training requirements |  |  |
|  | SAF202, SAF202A, SAF302, SAF303, SAF307 Depending on electrical hazards present, SAF603a or SAF104 may also be required. |  |  |
|  | - Unusual/Emergency procedures e.g., Injury, Fire, Loss of power |  |  |
|  | N/A |  |  |
|  | - Instrument calibration requirements, e.g., safety system/device recertification, RF probe calibration |  |  |
|  | N/A |  |  |
|  | - Inspection schedules |  |  |
|  | Inspection details & schedules of fall protection components are given on ESH&Q Manual Chapter 6131, Appendix T2. Note that all fall protection the must be inspected by workers before to and after each use. Aerial platform inspection is described on ESH&Q Chapter 6140, Appendix T3. |  |  |
|  | - References/Associated Documentation |  |  |
|  | N/A |  |  |
|  | - List of Records Generated (Include Location / Review and Approved procedure) |  |  |
|  | N/A |  |  |

- Authorized/Trained Individuals

| Print Name/Signature | Date |
| --- | --- |

|  | ISSUING AUTHORITY | APPENDIX AUTHOR | APPROVAL DATE | EFFECTIVE DATE | EXPIRATION DATE | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ESH&Q Division | Harry Fanning | 10/05/09 | 01/01/10 | 10/05012 | 0 |  |  |

This document is controlled as an on line file.  It may be printed but the print copy is not a controlled document.  It is the user’s responsibility to ensure that the document is the same revision as the current on line file.  This copy was printed on 7/28/2026.