|  | 3210Appendix T2 Task Hazard Analysis (THA) Worksheet |  |
| --- | --- | --- |

| Author: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: | June 16, 2011 |  |  |  |  | Task #: If applicable |  |  |  |  | Frequency of use: |  |  | Routine |
| Complete all information. Use as many sheets as necessary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Task Location: |  |  |  | Site wide |  |  |  |  | Task Title: |  |  | Standard work requiring personal fall protection |  |  |
| Division: |  |  | Physics |  |  |  |  |  |  | Department: |  |  |  |  |
| Lead Worker: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mitigation already in place: Standard Protecting Measures Work Control Documents |  |  |  |  |  |  | > EHS&Q Ch 6131 & 6132 > Personal Fall Protection Training and Equipment > Harness Fitting > Harness Inspection prior to use. |  |  |  |  |  |  |  |

| Sequence of Task Steps | Task Steps/Potential Hazards | Consequence Level | Probability Level | Risk Code (before mitigation) | Proposed Mitigation (Required for Risk Code >2) | Safety Procedures/ Practices/Controls/Training | Risk Code (after mitigation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Working from elevated areas with designated /approved anchor points or from platforms, scaffolds, aerial lifts, scissor lift where worker’s feet are over 4 feet above ground level | M | M | 3 | > OSP > Proper training & execution of approved procedures | > Fall Protection Harness to be worn. > Properly designed and approved anchor points > Correct length lanyard > Temporary guard rails. > Ladder Safety EHS&Q Ch 6132 > Ladder safety Training SAF307 > Aerial Platform & Fall Protection EHS&Q Ch 6147& 6131 > Aerial Platform Training SAF302 > Personal Fall Protection Training-SAF202 > Harness Fitting - SAF202A 11. > Harness Inspection prior to use. EHS&Q Ch 6131-T2-6.1 > Harness stored as per recommendations in EHS&Q Ch 6131-T2 - 6.2 | 1 |
|  | Crush injury resulting from operating lift in tight areas and getting caught between basket and fixed object/equipment | M | L | 2 | > OSP > Proper training & execution of approved procedures | > EHS&Q Ch 6147, 6131 & 6132 > Aerial Platform Training – SAF302 | 1 |
|  | Gaining access to work location & set up for man lift, scissor lift and scaffolding applications | M | M | 3 | > OSP > Proper training & execution of approved procedures >Ensure ground will support equipment weight, is stable & level > Equipment suitable for the terrain | > EHS&Q Ch 6147, 6131 & 6132. SAF 202, 302, 307 > Aerial Platform Training -SAF302 | 1 |
|  | Access work location via ladder | M | M | 3 | > OSP > Proper training & execution of approved procedures > Ensure ground will support equipments weight, is stable & level. | > EHS&Q Ch 6132 T1 > SAF307 | 1 |
|  | Unqualified electrical worker in an elevated position less than 10 feet from energized electrical conductors (>50 amps) OR (>50 volts AND >5 mA) AC or DC. | H | M | 4 | Training Use ladders made from fiberglass or wood if working near electrical hazards Observe other equipment/activities in the area that may pose a hazard Observe work areas for electrical hazards OR De-energize nearby electrical equipment | > SAF 603a Electrical Safety Awareness > EHS&Q Ch 6132 Appendix T1-Ladders > EHS&Q Ch 6131 Fall Protection Systems > EHS&Q Ch 6200 Jefferson Lab’s Electrical Safety Program OR > EHS&Q Ch 6110 Lock, Tag, Try (LTT) Program > SAF104 – Lock, Tag, and Try | 1 |

|  | Highest Risk Code before Mitigation: | 4 |  | Highest Risk Code after Mitigation: | 1 |
| --- | --- | --- | --- | --- | --- |

When completed, if the analysis indicates that the Risk Code before mitigation for any steps is “medium” or higher (RC≥3), then a formal Work Control Document (WCD) is developed for the task.  Attach this completed Task Hazard Analysis Worksheet.  Have the package reviewed and approved prior to beginning work.  (See ES&H Manual Chapter 3310 Operational Safety Procedure Program.)

|  | ISSUING AUTHORITY | FORM TECHNICAL POINT-OF-CONTACT | APPROVAL DATE | EFFECTIVE DATE | EXPIRATION DATE | REV. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ESH&Q Division | Harry Fanning | 10/05/09 | 01/01/09 | 10/05/12 | 0 |  |  |

This document is controlled as an on line file.  It may be printed but the print copy is not a controlled document.  It is the user’s responsibility to ensure that the document is the same revision as the current on line file.  This copy was printed on 7/28/2026.