| Jefferson Lab Alignment Group Data Transmittal |  |  |  |
| --- | --- | --- | --- |
| TO: S. Stepanyan |  | DATE: 31 Aug 2015 |  |
| FROM: J. Dahlberg | Checked: |  | # : B1666 |

DETAILS:									Data: 2B\HallB\HPS\Ecal\150828a

Below are the results from the recent survey carried out on the HPS Ecal reference points.  Values (in millimeters) are relative to the ideal pair spectrometer center.  A +X is to the beam left, a +Y is above, and a +Z is downstream.  Note that a 25.4 mm offset survey target was used to measure each point.  If coordinates of the ¼” hole on the reference point bushings are required, 1” would have to be subtracted from each Y coordinate.

LOCATION		     X		     Y		      Z

CTOP_A		-298.67	 233.70	  939.11

CTOP_B		-298.25	 233.86	1089.02

CTOP_C		 516.53	 233.26	  938.25

CTOP_D		 517.07	 233.24	1088.29

CBOTT_A		-299.94	-232.07	  937.16

CBOTT_B		-296.81	-232.44	1086.27

CBOTT_C		 517.21	-231.41	  935.97

CBOTT_D		 517.44	-231.45	1086.01