| Traveler Title | SNS PPU Cold Mass Assembly He Outlet Bellows and Con Bellows Weld Inspection Traveler |  |  |  |
| --- | --- | --- | --- | --- |
| Traveler Abstract | Traveler prescribes JLab weld inspection of the welds on the He Outlet Bellows and Con Bellows that are used on SNS PPU cryomodule cold mass assembly helium headers cryogenic circuit |  |  |  |
| Traveler ID | SNSPPU-INSP-HOBCB-WELD |  |  |  |
| Traveler Revision | R1 |  |  |  |
| Traveler Author | Gary Cheng |  |  |  |
| Traveler Date | 25-Sep-20 |  |  |  |
| NCR Informative Emails | Cheng, fischer, edaly |  |  |  |
| NCR Dispositioners | Cheng, fischer, edaly |  |  |  |
| D3 Emails | Cheng, fischer, edaly |  |  |  |
| Approval Names | G. Cheng | J. Fischer | J. Alston | E. Daly |
| Approval Signatures |  |  |  |  |
| Approval Dates |  |  |  |  |
| Approval Title | Author | Reviewer | JLab CWI | Project Manager |

| References | List and Hyperlink all documents related to this traveler. This includes, but is not limited to: safety (THAs, SOPs, etc), drawings, procedures, and facility related documents. |  |  |  |
| --- | --- | --- | --- | --- |
| 104211500-M8U-8200-A017 He Outlet Bellows | 104211500-M8U-8200-A026 Con Bellows | Kit - Part Serial Numbers for SNSPPU-CWI-HOBCB |  |  |

| Revision Note |  |
| --- | --- |
| R1 | Initial release of this Traveler. |

| Step No. | Instructions | Data Input |
| --- | --- | --- |
| 1 | A certified weld examiner/inspector is required. Inspector Name Date of Inspection | [[InspectorName]] <<SRF>> [[InspectionDate]] <<TIMESTAMP>> |
| 2 | The He Outlet Bellows and Con Bellows are kitted. Each kit consists of 2× He Outlet Bellows and 1× Con Bellows. Refer to Kit - Part Serial Numbers for SNSPPU-CWI-HOBCB for part SNs contained in kits. There is also spare Con Bellows that are not kitted. Either a kit or a spare bellows is under inspection at a time. Enter kit serial number (HOBCBSN): Or, spare Con Bellows serial number (HOBSN) Note: after inspection, parts shall resume kitted as they were. | [[HOBCBSN]] <<HOBCBSN>> [[HOBSN]] <<HOBSN>> |
| 3 | Vendor documents are uploaded to SharePoint folder CMASS_He Outlet & Con Bellows. Review vendor weld inspection report and comment. | [[VendorRptComment]] <<COMMENT>> |
| 4 | All bellows are required to be built in accordance with ASME B31.3. Inspect accessible welds exterior and interior to make sure that: - Welds are good, i.e. weld interior and exterior should be smooth and flush, with no crevices, cracks or protrusions, weld oxidation/sugaring shall have been brushed off. Verify weld sizes and patterns meeting specification in drawings. Make notes of any nonconformances. - There should be no unusual discoloration to the base material, especially around the welds. Generate NCR per weld inspector’s judgement. | [[WeldsOk]] <<YESNO>> [[ColorationOk]] <<YESNO>> [[WeldsComments]] <<COMMENT>> |
| 5 | Upload weld inspection report and pictures of failed areas or areas of concern. | [[WeldSurfPics]] <<FILEUPLOAD>> [[Weld_Insp_Rpt]] <<FILEUPLOAD>> |