|  | Welding and Brazing Final Visual Inspection (See ES&H Manual Welding and Brazing Program Supplement for more information.) |
| --- | --- |

| Final Visual Inspection – INFORMATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group: SNS PPU |  |  | Component: Helium Vessel – Shell Weldment |  |  |  |  | Date: 9/25/20 |
| Drawing #: 1042211700-M8U-8200-A020 |  |  | Rev#: |  | PQR #: On File |  |  | WPS/BPS #: On File |
| Welder/Brazer #: Vendors Welder (On File) |  |  | Process: GTAW |  |  | B31.3 Fluid Service: N/A |  |  |
| Acceptance Criteria |  | ASME B31.3 Chapter VI [Table 341.3.2] |  | AWS D1.1 Chapter VI [Table 6.1] |  |  | Other Code(s): ASME Section VIII- Div. 1 |  |
| # of Welds or Brazes Examined: SHELL WELD ONLY – SN: AA, AN, AP, AR, AT, AU, AV |  |  | # of Mechanical Joints Examined: N/A |  |  |  |  |  |

| Final Visual Inspection – CHECKLIST |  |  |  |
| --- | --- | --- | --- |
|  | Accept | Reject | N/A |
| - Mechanical joints have been properly assembled |  |  |  |
| - Mechanical joints have been properly erected including alignment supports and cold spring |  |  |  |
| - Welds/Brazes were sufficiently cleaned as not to impair visual inspection and are uniquely identified |  |  |  |
| - Geometry or profile conforms to WPS/BPS, drawing details or Code requirements |  |  |  |
| - Weld joint was properly identified to welder/brazer |  |  |  |
| - All flux/slag removed |  |  |  |
| WELDING ONLY: |  |  |  |
| - Weld has no visible cracks |  |  |  |
| - Thorough fusion (weld and base metals) and complete penetration if required exist |  |  |  |
| - Surface porosity or exposed slag inclusion |  |  |  |
| - Surface finish |  |  |  |
| - Concave root surface (suck up) or lack of weld joint break down |  |  |  |
| - Internal and external reinforcement within allowable limits |  |  |  |
| - Any fillet weld undersize (AWS only) within limits and undercutting depths |  |  |  |

| REMARKS: |  |
| --- | --- |

| SIGNATURE |  |  |  | DATE | 9/25/20 |
| --- | --- | --- | --- | --- | --- |
|  |  | EXAMINER / INSPECTOR |  |  |  |