| Upstream Feed Cap Installation Procedure for LCLS-II Cryomodules |  |  |  |
| --- | --- | --- | --- |
| Document Number: | L2HE-PR-CMA-FDCP-INST | Effective Date: | 27 Aug 2024 |
| Revision Number: | R1 | Periodic Review Date: | 27 Aug 2025 |
| Document Owner: | J. Fischer | Department Owner: | SRF Operations |

# Purpose

The purpose of this document is to describe the installation of the upstream feed cap for LCLS-II/HE Cryomodules

# Scope

This procedure applies to Feed Cap for LCLS-II Cryomodules and shall be performed by trained Cryomodule Assembly Technicians only.

# Terms and Definitions

The following terms have specific meanings within this procedure.

| Term | Definition |
| --- | --- |
| Feed Cap | Used for Cryomodule shipping and to support the GHRP. |
| GHRP | The supporting structure for the cavity string, gaseous helium return pipe, part of the Upper Cold Mass |
| Spindle | Inner mandrel, link between shipping insert and End cap |
| Shipping Insert | Assembly which is installed into the GHRP end to support the cavity string during transit |

# Roles and Responsibilities

The following roles have responsibilities described in this document.

| Role | Responsibility |
| --- | --- |
| Cryomodule Assembly Technician | Is trained and executes this Procedure performing described mechanical tasks |
| Cryomodule Assembly Lead/SME | Overlooks the execution of this Procedure and documents the results and any lessons learned |

# Procedure

## Preliminary Notes

### Installation of the Feed Cap should only be completed after the End Cap has been installed

### Ensure that the SPINDLE PUSH ROD (F10054584) has anti-seize applied to its threads prior to installation into the PLUG THREADED COLLAR (F10054537)

## Hardware List

| Description | Material | Quantity |
| --- | --- | --- |
| SG1 |  |  |
| ½-13 UNC x 5-1/2LG SHCS | Alloy Steel | 24 |
| M12 x 1.75 x 110mm LG HHCS | 316 Stainless Steel | 24 |
| M12 Flat Washer | Zinc Y-C Grade-8 Steel | 24 |
| ½” Flat Washers | Zinc Y-C Grade-8 Steel | 48 |
| ½” Bellville Washer | 18-8 Stainless Steel | 48 |
| SG2 |  |  |
| 3/8-16 UNC x 1-3/4LG SHCS | Black Oxide Alloy Steel | 12 |
| 1/3” Flat Washer | Zinc Y-C Grade-8 Steel | 12 |
| 5/16-18 UNC x 1-1/2LG HHCS | 18-8 Stainless Steel | 4 |
| 5/16” Flat Washer | Zinc Y-C Grade-8 Steel | 4 |
| SG3 |  |  |
| 3/8-16 UNC x 1-1/4LG SHCS | Grade-5 Steel | 16 |
| 3/8-16 UNC x 3-1/4LG SHCS | Grade-5 Steel | 8 |
| 3/8-16 UNC x 4-1/2LG HHCS | Grade-5 Steel | 1 |
| 3/8” Split Lock Washer | 18-8 Stainless Steel | 16 |

## Assembly Overview

### Sub-Group 1 (SG1): Feed Cap

### Sub-Group 2 (SG2): HeGRP Attachment, shipping insert

### Sub-Group 3 (SG3) Spindle Push Rod Cover

## SG2 Assembly

## Step 1: Press Plates of SG2

### Insert the two Press Plates on the GHRP in the vertical orientation

### Flat surfaces should be at 12 and 6 o’clock positions

### Outer lip of Press Plate should be outside GHRP rim

## Step 2: Install Centering of SG2

### Insert Centering Ring between Press Plates

### Tabs on the Centering Ring should be aligned Horizontally

## Step 3: Install Bottom Slide Ring of SG2

### Insert Slide Ring on Bottom Half of GHRP

### Tab on the Slide Ring should be at 6 o’clock position

### Tab should fit between the Press Plates

## Step 4: Install Top Slide Ring of SG2

### Insert Slide Ring on Top half of GHRP

### Tab on the Slide Ring should be at 12 o’clock position

### Tab should fit between the Press Plates

## Step 5: Position Front Ring of SG2

### Place Front Ring on SG2

### Hold Front Ring in place using two 3/8-16 SHCS and washers

## Step 6: Fix screws of SG2

### Fix the Front Ring in place using the remaining ten 3/8-16 SHCS, along with washers

### Finger tighten only

### Make note if the assembly does not allow all the screws to be installed

## Step 7: Torque screws of SG2

### Torque screws to 5 ft/lbs in a star pattern, repeat three times

## Step 8: Completed SG2 Installation

### Ensure that the assembly does not axially rotate.

## Step 9: Setup of SG1

### Ensure the parts and hardware for the Feed Cap (F10054678) are all available

### Apply Anti-Seize to the Spindle Push Rod threads

### Ensure the O-rings on the CM vacuum vessel are in place

## Step 10: Assembled SG1

### Assemble Feed Cap as per F10054678

### Ensure that the Glacier Plate (Item 6) is in place

## Step 11: Remove Spindle Push Rod Cover from SG3

### Remove the Spindle Push Rod Cover

## Step 12: Adjust SG3 Spindle Push Rod

### Adjust the Spindle Push Rod until it is 4” – 5” from the base of the cap.

### Hand tighten the SHCS to ensure that the Feed Cap Spindle is coupled to the Spindle Push Rod

## Step 13: Lift SG1 Assembly

### Attach clevis with sling and scale to the Feed Cap lifting lug

### Lift the Feed Cap to the approximate height of the GHRP

### Note the weight on the scale; it should read ~900lb. It may vary by 10%

## Step 14: SG1 Height Setting

### Measure the distance from the center of the GHRP to the ground

### Adjust the height of the Feed Cap until the center of the spindle is at an equal height from the ground

### Use a long level rule to ensure that the spindle is aligned to the center of the HeGRP Insert (SG2)

## Step 15: SG1 Positioning

### Carefully bring the Feed Cap towards the vacuum vessel until the flanges meet.

### If the spindle comes into contact with the HeGRP Insert first, turn back the Spindle Push Rod

### Monitor the scale; if the weight increases, it indicates that the spindle has hit the top of the insert and the cap needs to be lowered, and vice-versa.

## Step 16: Install Flange Hardware

### Turn the rotatable flange on the vacuum vessel until the holes in both the inner and outer bolt pattern line up. Mark this orientation

### Install the ½-13 UNC Alloy Steel SHCS with a flat washer on both the head and nut side, and two Bellville washers in a series on the head side.

### Use a single flat washer with each of the M12 x 1.75 SS HHCS

## Step 17: Spindle Push Rod Positioning

### Turn the Spindle Push Rod clockwise by hand to move the spindle into the HeGRP Insert. Stop once there is resistance.

### Tighten the SHCS by hand

### Using the hex feature, torque the Spindle Push Rod to 74 ft.lbs

## Step 18: Spindle Push Rod Lock

### Use the 3/8-16 UNC x 4-1/2” HHCS to lock the Spindle Push Rod in Place

## Step 19: Install Cover

### Reinstall the Spindle Push Rod Cover

### Use a single split lock washer for each 3/8-16 UNC SHCS

### Torque the SHCS to 31 ft.lbs

# References

| Document No. | Title |
| --- | --- |
| SRF-01-ML-001 | SRF Quality Manual |

# Release and Revision History

| Rev # | Major Changes | Effective Date: |
| --- | --- | --- |
| 1 | Initial version, added to Procedure template | 27 Aug 2024 |

# Approvals

| Approved by: | Name: | Signature: | Date: |
| --- | --- | --- | --- |
| Document Owner | John W. Fischer | In Docushare |  |
| Subject Matter Expert | Jared Martin | In Docushare |  |
| Work Center Lead | John W. Fischer | In Docushare |  |
| Project Leader | Adam Grabowski | In Docushare |  |